3GPP TSG-SA3 Meeting #112 	S3-233938

# Gothenburg, SE, 14 - 18 August 2023											(revision of S3-yyxxxx)

# Source:	Ericsson, Interdigital, Nokia, Nokia Shanghai Bell

# Title:	Consolidated Padding Solution: Concealing length of SUPIs in SUCIs by padding the SUPIs

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approval is requested.

# 2	References

# 3	Rationale

The solution is consolidated from Solutions #2, #5, and #9 based on the decision made on SA3#111.

# 4	Detailed proposal

*** 1st CHANGE ***

## 6.X	Solution #X: Concealing the length of SUPIs in SUCIs by padding the SUPIs (consolidated from Sol #2, 5, and 9)

### 6.X.1	Introduction

This is a solution to Key Issue #1: Privacy aspects of variable-length user identifiers. The key issue states that some network operators may decide to allow user identifiers with variable length, e.g., in the case of NAI-format SUPI. The length can become visible to an attacker in case a length-preserving encryption scheme is used to conceal the variable-length identifiers. If an attacker can learn the length of the identifier, such knowledge will reduce the size of the anonymity set.

The solution supports the following authentication mechanisms: 5G-AKA, EAP-AKA', and EAP-based authentication methods such as EAP-TLS and EAP-TTLS.

In the cases of 5G AKA and EAP-AKA', the solution uses pre-encryption padding (and post-decryption un-padding) of SUPIs before (and after) using the existing SUPI protection schemes (such as Profile A and Profile B, which are ECIES-based) as described in Annex C of TS 33.501 [aa].

In the cases of EAP-based methods, such as EAP-TLS and EAP-TTLS, padding is done to the SUPI or the TLS record that carries the TLS certificate with the NAI-format SUPI.

Padding of SUPIs is performed in the UE and un-padding in the UDM/SIDF. Padding and un-padding of the TLS record is done by the UE’s and the network’s (AMF/SEAF) TLS implementation.

### 6.X.2	Solution details

#### 6.X.2.1 Solution Overview

The overall solution comprises of the following parts:

- The operator provisions the padding parameters to the UE. The padding parameters include all the details about how the padding and un-padding of SUPIs are to be performed.

- Padding and un-padding of the NAI-format SUPI or the TLS record carrying the TLS certificate with NAI-format SUPI.

Figure 6.X.1-1 shows an overview of the solution with a flow diagram.

Figure 6.X.1-1: Solution overview

NOTE 1: 	SUPI in IMSI format is not padded because it is fixed length, and SUCIs cannot be linked with a particular SUPI based on length.

#### 6.X.2.2	Padding parameters

Padding parameters are chosen by the home network operator and provisioned to the UE. The UE stores the padding parameters. The padding parameters comprise of the following information elements that the UE uses to pad. Padding parameters can be assigned per subscriber basis, i.e., for one information element, the operator may choose different values for different subscribers.

- List of pLen:

-	It indicates padding length.

-	Each pLen in the list indicates the number of octets to be padded with the unpadded username or TLS record for a certain number of octets in the unpadded username.

-	If a pLen is the pair (x, y) where x and y are integers, then the NAI-format SUPI’s username which has the length of x octets, will be padded by y number of padding characters.

-	If a pLen is the pair (x, y) where x is an integer and y refer to a string, namely "rand", then the NAI-format SUPI’s username which has the length of x octets, will be padded by a randomly chosen (using a random number generator) number of padding characters.

-	The HN stores the whole list of pLen. The UE stores only one element of the pLen that is relevant for the associated subscriber.

NOTE 2: If an operator wants to make the length of all the padded SUPIs the same z, then the pLen value for a subscriber will be (x, y = z-x) where x is the length of the subscriber’s NAI-format SUPI’s username. This can be called fixed-length padding. When the value of y is set to the string "rand", the padding can be called random-length padding. If the operator sets different values to y for different subscribers based on some other preferred logic, then the padding can be called variable-length padding.

- Padding location (optional): Possible values are (i) prepending (ii) appending

- Padding delimiter (optional): Any non-UTF-8 characters (see RFC 3629 [4]), which are not allowed to be used for an NAI format SUPI, can be chosen as the padding delimiter.

- Padding character (optional):  Any non-UTF-8 characters (see RFC 3629 [4]), which is not allowed to be used for NAI and is not used as the padding delimiter, can be chosen as the padding character.  If no padding character is configured, any non-UTF-8 character can be used for padding.

- When additional flexibility of padding and unpadding is needed, such functionality may be supported by "padding method" parameter, that may be provisioned to the UE and/or transmitted to the UDM/SIDF inside protected SUCI.

#### 6.X.2.3	Padding and Un-padding

##### 6.X.2.1.1	5G-AKA and EAP-AKA':

In NAI-format SUPIs, the subscription identifier part includes the "username" portion of the NAI as defined in NAI RFC 7542 [5].

While computing a SUCI (using the ECIES scheme as described in Annex C of TS 33.501 [3]), with an exception for the null scheme, the UE pads the username portion before encrypting it using ECIES. Figure 6.X.2-1 presents the steps, which are also described below:

Figure 6.X.2-1: Initiation of 5G AKA and EAP-AKA' using SUCI generated from padded NAI-format SUPI

0.  Home network operator provisions padding parameters to the UE.

1.	Padding of SUPIs in NAI format is performed by the same component, either USIM or ME, which performs the calculation of SUCI in the following manner based on the provisioned padding parameters:

-	If a pLen indicates the pair (x, y) where x and y are integers, then the NAI-format SUPI’s username which has the length of x octets, will be padded by y number of configured padding characters.

-	If a pLen is the pair (x, y) where x is an integer, and y refers to a string, namely "rand", then the NAI-format SUPI’s username, which has the length of x octets, will be padded by a randomly chosen (using a random number generator) number of padding characters.

-	The padding is done by appending or prepending (according to the padding location configured in the padding parameters) the padding characters. If padding location is not present in the padding parameters, the UE can choose either prepending or appending.

-	If a padding character is present in the padding parameters, the UE uses the padding character. Otherwise, the UE can choose any character that is not allowed to be part of a valid NAI-format SUPI username.

-	If a padding delimiter is present in the padding parameters, then the delimiter will be inserted before (when appended) or after (when prepended) adding the padding characters.

- The UE does not pad the SUPI in NAI format when using the null scheme.

NOTE 3: 	The null scheme does not provide SUPI privacy; therefore, padding will only increase message length without improving any privacy.

2. The UE performs ECIES-based encryption (as described in Annex C.3.2 of TS 33.501 [aa]) on the resulting padded username to generate the ciphertext used to form the final SUCI output.

3. The UE sends the resulting SUCI to the network

4. The SEAF forwards the SUCI containing SUPI in NAI format to the AUSF

5. The AUSF forwards the SUCI containing SUPI in NAI format to the UDM/SIDF

6. The UDM/SIDF performs ECIES-based decryption of the ciphertext part of the SUCI to deconceal padded SUPI in NAI format as per TS 33.501 [aa].

7. From the resulting cleartext padded username, UDM/SIDF filters out special characters that cannot be used for a username based on IETF RFC 7542 [5] and RFC 3629 [4] (i.e., not a UTF-8 character set) to obtain the unpadded username part of the SUPI.

##### 6.X.2.1.2	EAP-TLS and EAP-TTLS:

In EAP-TLS and EAP-TTLS, an anonymous SUCI (instead of a SUCI that is computed using an ECIES scheme) can be sent first, and the actual SUPI is sent (possibly as part of the UE’s TLS certificate) after an EAP secure channel is established, e.g., the TLS tunnel. In these cases, the proposed solution performs the padding and un-padding in the following manner:

- when the UE sends the SUPI (which is not concealed in a SUCI) over the TLS channel (but not part of the UE’s TLS certificate), the UE pads the SUPI in a similar manner (Step 1 in Clause 6.X.2.1.1 of this solution) as it does before encrypting using ECIES –.

- When the SUPI is sent as part of a TLS certificate, the UE pads the TLS record that is carrying the certificate.

  - If a pLen indicates the pair (x, y) where x and y are integers, then the TLS record that carries a TLS certificate with NAI-format SUPI, which has the username of the length of x octets, will be padded by y number of characters. The padding will be done inside the TLS implementation that takes y as input.

  - If a pLen indicates the pair (x, y) where x refers to an integer, and y refers to a string, namely "rand", then the TLS record that carries a TLS certificate with NAI-format SUPI, which has the username of the length of x octets, will be padded by a randomly chosen (using a random number generator) number of padding characters. The padding will be done inside the TLS implementation that takes the randomly chosen number as input.

- The network does not need to do anything to unpad the TLS record because the TLS implementation itself provides an unpadded TLS record on the network side.

### 6.X.3	Evaluation

This solution addresses KI #1.

It solves the problem of leaking SUPI length in the context of 5G AKA, EAP-AKA', EAP-TLS, and EAP-TTLS using padding.

If the padding parameters are chosen in such a way that all the padded SUPIs have the same length or randomly chosen length, then the length of SUCIs or TLS records does not leak any information about the concealed SUPI.

The UE has to be provisioned with padding parameters. SUPI padding needs to be done by the USIM or ME, depending on where the SUCI is computed. The UDM/SIDF has to unpad the padded SUPI.

The unpadding of the TLS record has no impact on the network side – it is handled as part of the TLS implementation.

The solution addresses the following:

-	If the UE is not provisioned with padding-related information, then no padding will be done. The network does not need to unpad.

-	If the USIM is provisioned with the padding-related information, the SUCI is computed in the ME, and the ME is not aware of any padding, then no padding will be done. The network does not need to unpad anything.

-	The following condition can never happen: the UE pads when the network has not provisioned the UE with padding configurations.

*** END OF CHANGES ***