3GPP TSG-SA3 Meeting #112 	S3-233953

Goteborg, Sweden, 14 - 18 August 2023

Title:	Rely LS on AKMA service restrictions in Rel-17

Response to:	         LS S3-233509 LS on AKMA service restrictions in Rel-17

Release:	Rel-18

Work Item:	AKMA-CT

Source:	China Mobile (to be SA3)

To:	CT3

Cc:	SA2

Contact person:	Xiaoting Huang

huangxiaoting@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA3 thanks CT3 for their LS on AKMA service restrictions in Rel-17, SA3 would like to answer CT3’s questions as below.

Question1: Is Rel-17 AKMA services supported for the UE connected via Non-3GPP access?

Answer: AKMA services support UE connection via both 3GPP access and non-3GPP access. In other words, the AKMA service is independent of UE access types.

Question2: If the answer to Question 1 is affirmative, then how the network could identify the PDU session is related to roaming UE and deny AKMA services?

Answer: Firstly, when the UE establishes PDU session via Non-3GPP access, HPLMN AMF/SMF is selected, and PDU session is treated as non-roaming, SA3 thinks this scenario is a non-roaming case as HPLMN AMF/SMF is selected, thus there is no issue that UE can connect to the AF in the HPLMN to access AKMA services via the Non-3GPP access.

Secondly, SA3 would like to point out in Rel-17 TS 33.535, the NOTE of roaming says “Roaming aspects are not considered in the present document” rather than “roaming is not supported”. For further information, based on SA3’s study in Rel-18 about AKMA roaming, it’s concluded that the Rel-17 AKMA architecture and procedures could support roaming scenarios without additional changes. Therefore, SA3 would like to request CT3 to consider whether it is appropriate for the network to deny AKMA services if it identifies the PDU session is related to roaming UE in Rel-17, considering the AKMA roaming aspects have been clarified in Rel-18.

2	Actions

To SA3-LI group

ACTION: 	SA3 kindly asks CT3 to take the above answers into consideration.

3	Dates of next TSG SA WG 3 meetings

SA3#113	6 -10 November 2023	Chicago, US

SA3#114	22 -26 January 2024	UE (TBD)