3GPP TSG-SA3 Meeting #112 	S3-233955

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.537 | CR | 0005 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections to TS 33.537 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_AAnF |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Editorial corrections, several references are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix the missing references |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | References are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***	START OF CHANGE	***

4.2.2	Security functional requirements on the AAnF deriving from 3GPP specifications and related test cases

4.2.2.0	General

The general approach in TS 33.117 [2] clause 4.2.2.1 and all the requirements and test cases in TS 33.117 [2] clause 4.2.2.2 related to SBA/SBI aspects apply to the AAnF network product class.

4.2.2.1	AKMA key storage and update

Requirement Name: AKMA key storage update

Requirement Reference: TS 33.535 [4], clause 6.1

Requirement Description: The AAnF stores the latest information sent by the AUSF. After receiving the new generated A-KID and KAKMA, the AAnF deletes the old A-KID and KAKMA and stores the new generated A-KID and KAKMA as specified in TS 33.535 [4], clause 6.1.

Threat References: TR 33.926 [6], Annex W.2.2.3, AKMA key storage and update

Test Case:

Test Name: TC_AKMA_Key_Storage_Update

Purpose:

Verify that the AAnF stores only the latest AKMA context received by the AUSF.

Pre-Conditions:

-	Test environment with AUSF and AF. The AUSF and the AF may be simulated.

-	AAnF network product is connected in emulated/real network environment.

Execution Steps

Test A:

1)	Primary authentication is simulated for a specific UE, leading to the simulated AUSF pushing SUPI, A-KID1, KAKMA1 to the AAnF.

2)	The AF requests a KAF from the AAnF by proving A-KID1 and AF_ID.

3)	Another primary authentication is simulated for the same UE, leading to the simulated AUSF pushing SUPI, A-KID2, KAKMA2 to the AAnF.

4)	The AF requests a KAF by providing A-KID1 to the AAnF.

5)	The AF requests a KAF by providing A-KID2 to the AAnF.

Expected Results:

The AF received an error message indicating the AKMA context related to A-KID 1 is not found after step 4). After step 5), the AF received a KAF which is different from the KAF that received after step 2).

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

***	START OF NEXT CHANGE	***

4.2.3.2	Protecting data and information

4.2.3.2.1	Protecting data and information – general

There are no AAnF-specific additions to clause 4.2.3.2.1 of TS 33.117 [2].

4.2.3.2.2	Protecting data and information – unauthorized viewing

There are no AAnF -specific additions to clause 4.2.3.2.2 of TS 33.117 [2].

4.2.3.2.3	Protecting data and information in storage

There are no AAnF -specific additions to clause 4.2.3.2.3 of TS 33.117 [2].

4.2.3.2.4	Protecting data and information in transfer

4.2.3.2.4.1	Confidentiality, integrity and replay protections over SBA interface

Requirement Name: Confidentiality, integrity and replay protections over SBA interface

Requirement Reference: TS 33.535 [4], clause 4.4.0

Requirement Description: The SBA interface between the AAnF and the AUSF is confidentiality, integrity and replay protected as specified in TS 33.535 [4], clause 4.4.0

Threat References: TR 33.926 [3], Annex W.2.2.1, Control plane data protection with AUSF

Test Case:

Test Name: TC_PROTECT_SBA_AAnF_AUSF

Purpose:

Verify that the transported data between AAnF and AUSF are confidentiality, integrity and replay protected over SBA interface.

Pre-Conditions:

-	AAnF and AUSF network products are connected in simulated/real network environment.

-	Network product documentation containing information about supported TLS protocol and certificates is provided by the vendor.

-	Tester shall have access to the SBA interface between AAnF and AUSF.

Execution Steps:

The requirement mentioned in this clause is tested in accordance with the procedure mentioned in clause 4.2.2.2.2 of TS 33.117 [2].

Expected Results:

The user data transported between AAnF and AUSF is confidentiality, integrity and replay protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture or pcap traces.

4.2.3.2.4.2	Confidentiality, integrity and replay protections over SBA interface

Requirement Name: Confidentiality, integrity and replay protections over SBA interface

Requirement Reference: TS 33.535 [4], clause 4.4.0

Requirement Description: The SBA interface between AAnF and AF/NEF is confidentiality, integrity and replay protected as specified in TS 33.535 [4], clause 4.4.0

Threat References: TR 33.926 [3], Annex W.2.2.2, Control plane data protection with AF/NEF

Test Case:

Test Name: TC_PROTECT_AAnF_AF_NEF

Purpose:

Verify that the transported data between AAnF and AF/NEF are confidentiality, integrity and replay protected over SBA interface.

Pre-Conditions:

-	AAnF and AF/NEF network products are connected in simulated/real network environment.

-	Network product documentation containing information about supported TLS protocol and certificates is provided by the vendor.

-	Tester shall have access to the SBA interface between AAnF and AF/NEF.

Execution Steps:

The requirement mentioned in this clause is tested in accordance with the procedure mentioned in clause 4.2.2.2.2 of TS 33.117 [2].

Expected Results:

The user data transported between AAnF and AF/NEF is confidentiality, integrity and replay protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture or pcap traces.

***	END OF CHANGE	***