3GPP TSG-SA3 Meeting #112 	S3-234028

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0175 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IETF OSCORE as AKMA Ua* protocol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_GBA_OSCORE |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | AKMA does not have any Ua* protocol specification for constrained IoT devices supporting Open Mobile Alliance (OMA) procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a normative annex that specifies the IETF OSCORE as Ua* protocol for constrained IoT devices supporting Open Mobile Alliance (OMA) procedures and as a potential Ua* protocol for MSGin5G (5GMSG) work item. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AKMA will not be used by IoT UEs which support OMA procedures. The 5GMSG feature will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, Annex Z(new), Annex Z.1 (new), Annex Z.2 (new), Annex Z.2.1 (new), Annex Z.2.2 (new), Annex Z.2.3 (new), Annex Z.2.4 (new), Annex Z.3 (new), Annex Z.3.1 (new), Annex Z.3.2 (new), Annex Z.3.3 (new), Annex Z.3.4 (new), Annex Z.3.5 (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System".

[4]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[5]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[6]	IETF RFC 7542: "The Network Access Identifier".

[7]	3GPP TS 33.222: " Generic Authentication Architecture (GAA); Access to network application functions using HypertextTransfer Protocol over Transport Layer Security (HTTPS)".

[8]	Void

[9]	3GPP TS 23.003: "Numbering, addressing and identification".

[10]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[11]	3GPP TS 29.503: "5G System; Unified Data Management Services ".

[XX]	IETF RFC 8613: "Object Security for Constrained RESTful Environments (OSCORE)"

[XY]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)"

[XZ]	IETF RFC 8949: "Concise Binary Object Representation (CBOR)"

[XW]	IETF RFC 8152: "CBOR Object Signing and Encryption (COSE)"

[ZZ]	IETF RFC 5869: "HMAC-based Extract-and-Expand Key Derivation Function (HKDF)"

*** 2nd CHANGE ***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

A-KID	AKMA Key IDentifier

A-TID	AKMA Temporary UE IDentifier

AAnF	AKMA Anchor Function

AF	Application Function

AF_ID	AF Identifier

AKMA	Authentication and Key Management for Applications

AMF	Access and Mobility Management Function

AUSF	AUthentication Server Function

CBOR	Concise Binary Object Representation

CoAP	Constrained Application Protocol

KAF	AKMA Application Key

KAKMA	AKMA Anchor Key

KDF	Key Derivation Function

NEF	Network Exposure Function

OSCORE	Object Security for Constrained RESTful Environments

RID	Routing InDicator

UDM	Unified Data Management

*** 3rd CHANGE ***

# Annex Z (normative): Ua* security protocol: Object Security for Constrained RESTful Environments (OSCORE)

## Z.1	General

This annex describes how to secure access to an AF using Object Security for Constrained RESTful Environments (OSCORE) [XX].

The specification of the OSCORE as an AKMA Ua* protocol follows the architecture of GBA OSCORE Ua protocol in TS 33.220 [4], Annex Y with the AF taking the role of the NAF.

## Z.2	Requirements

### Z.2.1	General

This Annex covers the aspects specific to the AKMA Ua* protocol based on OSCORE. This feature is optional to be supported for the UE and AF. If the feature is supported, the following clauses apply.

### Z.2.2	Requirements on the UE

To utilise AKMA as described in this document the UE shall be equipped with an CoAP capable client implementing the particular features of AKMA as specified in this document.

### Z.2.3	Requirements on the AF

To utilise AKMA as described in this document the AF shall support the features of AKMA as specified in this document.

### Z.2.4	Requirements on the OSCORE

The same requirements outlined in TS 33.220 [4], clause Y.2.4 apply in this clause.

## Z.3	IETF OSCORE as an AKMA Ua* protocol

### Z.3.1	General

The IETF OSCORE as an AKMA Ua* protocol is specified in this clause by providing the details about the procedures, the OSCORE security context and how it is related to the AKMA KAF and the encoding of OSCORE messages using IETF CBOR specified in IETF RFC 8949 [XZ].

### Z.3.2	Procedures

The procedures for the AKMA OSCORE Ua* protocol are the same as the TS 33.220 [4], clause Y.3.2 with the following changes.

1)	In Step 1, the CoAP Client (UE) shall send a CoAP request to the AF. This is the Application Session Establishment Request in Step 1 in clause 6.2. The CoAP request shall consist of the following:

i)	CoAP Method: POST

ii)	URI of the AKMA resource on the AF. The URI shall have the format of <AF_IP_or_FQDN>/akma, where AF_IP_or_FQDN indicates the IP address or the FQDN of the host that hosts the AF.

NOTE 1: It is assumed that the AF IP address or FQDN is already provisioned to the UE for AKMA purposes.

iii)	Payload: CoAP Security protocol identifer, A-KID, N1, AF-SID, ?OSC-INP

The parameters "CoAP Security protocol identifier", N1, AF-SID, ?OSC-INP have the same semantics as the corresponding parameters in TS 33.220 [4], clause Y.2.3. Step 1.

2)	 Steps 2-4 follow clause 6.2 in this specification

3)	The CoAP Server (AF) shall respond to the CoAP Client (UE) with a CoAP response. This is the Application Session Establishment Response in Step 5 in clause 6.2. The response shall have the following content:

i)	Response Code: "Created"

ii)	Payload: N2, UE-SID

The parameters N2, UE-SID have the same semantics as the corresponding parameters in TS 33.220 [4], clause Y.2.3. Step 3.

### Z.3.3	OSCORE Security context

The OSCORE security context used in AKMA OSCORE Ua* protocol is similar to the GBA OSCORE security context specified in TS TS 33.220 [4], clause Y.3.3 with the following changes. The  OSCORE security context for the OSCORE profile of Ua* shall have the following values:

-	OMS = OSCORE Master Secret = HKDF(KAF, "AKMA-OSCORE")

-	Master Salt = Request Payload | Response Payload

-	UE Sender ID = UE-SID generated by CoAP Server and sent to the CoAP Client in the Application Session Establishment Response (Step 3 in clause Z.3.2)

-	AF Sender ID = AF-SID generated by CoAP Client and sent to the CoAP Server in the Application Session Establishment Request(Step 1 in clause Z.3.2)

where HKDF shall be the HMAC-based Key Derivation Function specified in IETF RFC 5869 [ZZ]

### Z.3.4	Refresh of OSCORE key material

OSCORE allows both the communication endpoints (UE or AF) to renegotiate the OSCORE security context after the OSCORE security context is established, according to Appendix B.2 in IETF RFC 8613 [XX], which is shown in the figure Z.3.4-1, Step 1.

Moreover even if KAF remains constant upon a new application session establishment (Step 1 in clause Z.3.2) or a renegotiation of the OSCORE key material, the nonces N1, N2, used in OSCORE security context shall be (stochastically) different from the previous OSCORE security context negotiation to ensure that the OSCORE security context is different.

Figure Z.3.4-1: OSCORE key refresh

### Z.3.5	OSCORE Ua* protocol payload encoding

IETF CoAP and OSCORE shall use the IETF Concise Binary Object Representation (CBOR) specified in the IETF RFC 8949 [XZ] for payload encoding for efficient information transfer between constrained IoT devices.

The CoAP media type for CBOR encoding shall be:

-	Media Type: application/cbor

-	CoAP Content-Format: 60

The Request Payload in the Application Session Request shall be formatted as a CBOR Array as follows:

Request Payload = [
A-KID : bstr,
      N1 : bstr, 
      AF-SID : bstr, 
      ? OSC-INP: bstr
]

A-KID = [
     RID : tstr,
     A-TID : bstr,
     HPLMN-ID : tstr
]

OSC-INP = {           ; CBOR Map
      ? 1 => int,	   ; version 
      ? 3 => int,	   ; hkdf 
      ? 4 => int,   ; alg 
      ? 5 => bstr, ; salt 
      ? 6 => bstr 	; contextId
}

The Response Payload in the Application Session Response shall be formatted as a CBOR Array as follows:

Reponse Payload = [         ; CBOR Array
	N2 : bstr, 
	UE-SID : bstr
]

*** END OF CHANGES ***