3GPP TSG-SA3 Meeting #112 	S3-234035

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.256 | CR | 0025 | rev | <Rev#> | Current version: | V17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to A2X Direct C2 Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The A2X Direct C2 Communication in Clause 5.Y.1, has the following EN: Editor's note: Enhancements to the UUAA (5.2) and/or Pairing (5.4) procedures related to unicast security establishment for Direct C2 are FFS. Further, Clause 5.Y.2 for Unicast mode direct C2 communication cites clause 5.X.2 for the procedures, wherein clause 5.X.2.3 Security procedure cites 33.536. The actual baseline procedure in TS 33.536 can be reused for A2X, but the aspects used in the procedure should be specific to A2X service rather than V2X service similar to TS 23.256 Clause 5.4.4 Procedure for Direct C2 Communication establishment, which has the following NOTE. NOTE: Security aspects of Direct C2 Communication is defined by SA WG3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolves the EN in Clause 5.Y.1 and clarifies the A2X service aspects for considering aspects in TS 33.536 as baseline. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The A2X security procedure will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.2, 5.Y.1 and 5.Y.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | n | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | n | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | n | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Change 1 ****

## 5.Y	A2X Direct C2 Communication

### 5.Y.1	General

The unicast mode Direct C2 Communication procedures are described in TS 23.256 [3]. Unicast mode Direct C2 Communication is used by two UEs that directly exchange traffic for the A2X applications running between the peer UEs.

Before taking part in Direct C2 Communication, the UAV needs to be authorised as described in TS 23.256 [3] (see also the present document for more details of C2 authorisation over the network).

The UUAA and UAV Pairing authorizaion with UAVC procedures described in clauses 5.2 and 5.3 respectively are reused with the direct C2 authorization specific adaptation described in TS 23.256 clause 5.4 and the following security specific adaptations:

- In case of Pairing authorization: If the authorization request for Direct C2 Communication was included in step 1 and the C2 authorization is successful, the USS may include direct C2 pairing information containing the UAV-C's Application Layer ID (as described in TS 23.256 and C2 session security information (for direct C2 security). The content of C2 session security information (e.g., key material to help establish security between the UAV and UAV-C) is not in 3GPP scope.

### 5.Y.2	Unicast mode Direct C2 Communication

Unicast mode Direct C2 Communication has the same requirements and procedures as unicast mode A2X Direct Communication (see clause 5.X.2 of the present specification) with the following exception:

-	The A2X Policy Provisioning is done based on TS 23.256 Clause 4.2.1.2.2 and the A2X Policy includes A2X security policy for each A2X services (e.g., C2 and DAA). The C2 service specific security policy available as part of A2X security policy is used for the security establishment, where the signalling and user plane confidentiality and integrity are set as required based on local policy.

The security procedure for unicast mode V2X communication described in TS 33.536 Clause 5.3.3.1.4.3 is reused with the following adaption for direct C2 communication.

- In step 1: The direct communication request includes UAV’s application layer ID (e.g., CAA-Level UAV ID), target UAV-C identifier (i.e., UAV-C’s application layer ID based on local configuration or if received during direct C2 authorization), A2X service type indication specific to C2 (i.e., which may be preconfigured in the UAV as described in TS 23.256 clause 5.4.4) and A2X security policy specific to C2.

- In step 2: The UAV-C checks the received UAV ID based on local configuration (which includes direct C2 pairing authorizations). If the check is successful rest of the procedure is continued, else it sends direct communication reject message that includes V2X service error with a suitable cause value.

- In step 3a: The Direct security mode command includes A2X security policy received in step 1 (e.g., to provide protection against bidding down attacks as described in TS 33.536).

- Throughout the procedure, V2X is replaced by A2X, e.g., V2X layer to A2X layer, V2X application layer to A2X application layer, etc.

**** End of Change 1 ****