3GPP TSG-SA3 Meeting #112	S3-234057

# Goteborg, Sweden 14 - 18 August 2023												   revision of S3-23xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | draft-CR | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security for integrated U2U relay discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-07-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Need a solution for Security for 5G ProSe UE-to-UE Relay Communication with integrated Discovery |  |  |  |  |  |  |  |  |
| Summary of change: |  | Security for 5G ProSe UE-to-UE Relay Communication with integrated Discovery |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete security specification for the UE-to-UE relay communication |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES************

6.6.5	Security for 5G ProSe UE-to-UE Relay Communication with integrated Discovery

There is a DCR security indicator in the DCR message sent by both source 5G ProSe End UE and 5G ProSe UE-to-UE Relay. The DCR security indicator indicates whether the DCR message is protected with discovery security materials that may be associated with ProSe service or RSC.

In the case of DCR message protection, the DCR message over the first hop PC5 reference point shall be protected as follows:

1.	If the source 5G ProSe End UE is configured with the discovery security materials associated with the ProSe service as specified in clause 6.1.3.2.3 of the present document, User Info ID of source 5G ProSe End UE and User Info ID of target 5G ProSe End UE shall be confidentiality protected with the DUCK associated with the ProSe service. Otherwise, the User Info ID of source 5G ProSe End UE and User Info ID of target 5G ProSe End UE is not confidentiality protected.

If there is a security policy that specifies the hop-by-hop security in the DCR message, the protection of this security policy shall be same as the User Info ID of source 5G ProSe End UE and User Info ID of target 5G ProSe End UE.

2.	If the source 5G ProSe End UE is configured with the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of the present document, RSC shall be confidentiality protected with the DUCK associated with the RSC. Otherwise, the RSC is not confidentiality protected.

3.	If the source 5G ProSe End UE is configured with the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of the present document, the DCR message shall be integrity protected with the DUIK associated with the RSC. Otherwise, the DCR message is not integrity protected.

In the case of DCR message protection, the DCR message over the second hop PC5 reference point shall be protected as follows:

1.	If the 5G ProSe UE-to-UE Relay is configured with the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of the present document, User Info ID of 5G ProSe UE-to-UE Relay and RSC shall be confidentiality protected with the DUCK associated with RSC. Otherwise, the User Info ID of 5G ProSe UE-to-UE Relay and RSC is not confidentiality protected.

2.	If the source 5G ProSe End UE is configured with the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of the present document, the DCR message shall be integrity protected with the DUIK associated with the RSC. Otherwise, the DCR message is not integrity protected.

************ END OF CHANGES************