3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	S3-234130

# Berlin, Germany, 22-26 May 2023														    was S3-23xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | draft | rev | - | Current version: | 17.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR to draft CR on resource owner aware northbound acces to APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPPY |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This pCR introduces client credential flow, PKCE flow and authorization revocation for SNAAPPY |  |  |  |  |  |  |  |  |
| Summary of change: |  | - client credential flow is introduced in clause 6.5.Y2e.x - PKCE flow is introduced in clause 6.5.Y2e.y - Authorization revocation may also be specified in TS23.222 (cf. first alternative in clause 6.5.Y2e.n) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource owner aware northbound access to APIs flows are undefined |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

++++++ First Change ++++++

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[3]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[4]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[5]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[6]	IETF RFC 7519: "JSON Web Token (JWT)".

[7]	IETF RFC 7515: "JSON Web Signature (JWS)".

[8]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[9]	Void

[10]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[XX]	IETF RFC 7636: " Proof Key for Code Exchange by OAuth Public Clients".

++++++ Next Change ++++++

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AEF	API Exposing Function

API	Application Programming Interface

CAPIF	Common API Framework

JSON	JavaScript Object Notation

JWT	JSON Web Token

KDF	Key Derivation Function

PKI	Public Key Infrastructure

PSK	Pre-Shared Key

RNAA	Resource owner-aware northbound API access

TLS	Transport Layer Security

++++++ Next Change ++++++

# 5	Functional security model

## 5.1	General functional security model

Figure 5-1 shows the functional security model for the CAPIF architecture. The interfaces CAPIF-1, CAPIF-1e, CAPIF-2, CAPIF-2e, CAPIF-3, CAPIF-4, CAPIF-5, CAPIF-3e, CAPIF-4e, CAPIF-5e, CAPIF-7 and CAPIF-7e are defined in 3GPP TS 23.222 [3] and support the CAPIF functionality defined in 3GPP TS 23.222 [3]. CAPIF-1, CAPIF-2, CAPIF-3, CAPIF-4, CAPIF-5 and CAPIF-7 are interfaces that lie within the PLMN trust domain while the CAPIF-1e , CAPIF-2e, CAPIF-3e, CAPIF-4e, CAPIF-5e and CAPIF-7e interfaces are CAPIF core and AEF access points for API Invokers outside of the PLMN trust domain.

Security for the CAPIF-1, CAPIF-2, CAPIF-3, CAPIF-4, CAPIF-5 and CAPIF-7 interfaces support TLS and are defined in subclauses 6.2, 6.4 and 6.6 of the present document. Security for the CAPIF-1e,  CAPIF-2e and CAPIF-7e interfaces support TLS, and are defined in subclause 6.3, subclause 6.5, and subclause 6.9 respectively.

Security for the CAPIF-3e, CAPIF-4e and CAPIF-5e interfaces support NDS/IP security to secure communication between different IP security domains. This avoids multiple secure connections between API provider domain and CAPIF core domain by leveraging the NDS/IP security procedures specified in TS 33.210 [2].

Authentication and authorization are required for both API invokers that lie within the PLMN trust domain and API invokers that lie outside of the PLMN trust domain. For an API invoker that is outside of the PLMN trust domain, the CAPIF core function in coordination with the API exposing function utilizes the CAPIF-1e, CAPIF-2e and the CAPIF-3 interfaces to onboard, authenticate and authorize the API invoker prior to granting access to CAPIF services. Security flow diagrams for onboarding security, CAPIF-1e security and CAPIF-2e security can be found in Annex B. When the API invoker is within the PLMN trust domain, the CAPIF core function in coordination with the API exposing function perform authentication and authorization of the API invoker via the CAPIF-1, the CAPIF-2 and the CAPIF-3 interfaces prior to granting access to CAPIF services. Authentication and authorization of API invokers (both internal and external to the PLMN trust domain) is specified in clause 6 of the present document.

Figure 5-1: CAPIF functional security model

## 5.2	Functional security model supporting RNAA

Figure 5.2-1 shows the functional security architecture of CAPIF when RNAA is supported.  The authorization function itself is a part of the CCF. The Oauth client and the CCF shall communicate using https.

Figure 5.2-1: CAPIF supporting RNAA functional security model

Editor's note: the above figure will need alignment with SA6.

++++++ Next Change ++++++

## 6.5	Security procedures for CAPIF-2e reference point

### 6.5.Y2e	Authentication and authorization for RNAA

#### 6.5.Y2e.1	General

The authorization for RNAA shall use token based authorization using oauth2 framework. The API invoker has the role of the OAuth client. The CCF has the role of the OAuth authorization server. The AEF has the role of the resource server.

The access tokens used for RNAA can contain the resource owner identity claim and other claims. Within the token, GPSI is used as identifier. To avoid privacy issues, GPSI needs to be different from MSISDN, SUPI etc. AEF should do the authorization check to the API invocation request containing the token against the claims including the resource owner identity. As the claim in the token includes resource owner identity, there is no need for additional UE authentication in API invocation. Moreover, the token should be able to restrict the API invoker to a specific resource (e.g., location, QoS, PDN connectivity status). For Oauth flows involving redirection, authentication between CCF/AUF and UE should be performed after API Invoker redirects the UE to CCF/AUF. Based on the operator's policy, mutual authentication between resource owner and CCF/AUF can be performed. The authentication method is up to operator policy (e.g. an already deployed mechanism, or AKMA, or GBA).

In case of an external AF (i.e., not the application on the UE) as API invoker, for mutual authentication of API invoker AF and API exposing function, TS 33.122 [5] is reused.

#### 6.5.Y2e.x	Authorization using oauth client credential flow

When no user interaction is required to get resource owner authorization, i.e. the authorization doesn't require user interaction, client credential flow according to RFC 6749 [4] shall be used. The CCF shall verify that, based on operator configurable policy, usage of client credential flow is acceptable for a particular scope and client-id. If it is not, the request shall be rejected. Otherwise, for this flow, Annex C shall apply with the following modifications:

-	In case of UE originated API calls, the client_id shall, in addition to an application identifier registered at onboarding, include a RNAA specific GPSI of the calling UE.

-	The scope shall contain the RNAA specific GPSI of the UE of the accessed resources.

-	Revocation shall be performed is described in 6.5.Y2e.n

#### 6.5.Y2e.y	Authorization using oauth PKCE flow

When user interaction is required to get resource owner authorization, PKCE flow according to RFC 7636 [XX] shall be used with the following profile:

-	Grant type shall be set to "PKCE"

-	The code verifier shall use the output of a cryptographic random number generator to create a 32-byte sequence. This sequence is then base64url-encoded to produce a 43 byte string to use as the code verifier

-	The code_challenge_method shall be "S256".

-	In case of UE originated API calls, the client_id shall, in addition to an application identifier registered at onboarding, include a RNAA specific GPSI of the calling UE.

-	The scope shall contain the RNAA specific GPSI of the UE of the accessed resources.

-	Revocation shall be performed is described in 6.5.Y2e.n

#### 6.5.Y2e.n	Revocation

API Exposing Function needs to be informed about revocation if this is necessary to ensure correct handling of revocation.

The revocation procedure is described in TS 33.122 [5].

[alternative]

Figure 6.5.Y2e.n -1 illustrates the procedure for revoking API invoker authorization to access service API initiated by the resource owner client.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

Figure 6.5.Y2e.n-1: Procedure for revoking API invoker authorization initiated by resource owner client

1.	The resource owner client triggers the revocation of the API invoker authorization to the CAPIF Core function.

2.	The CAPIF core function sends revoke API invoker authorization request to the AEF with the details of the API invoker and the service API.

3.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the AEF invalidates the API invoker authorization corresponding to the service API.

4.	The AEF sends a revoke API invoker authorization response to the CAPIF core function.

5.	The CAPIF core function invalidates the API invoker authorization corresponding to the service API.

6.	The CAPIF core function sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

+++++ End of Changes +++++