3GPP TSG-SA3 Meeting #112 	S3-234245

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.328 | CR |  | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex N additions for IMS data channels. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_SEC |  |  |  |  |  | Date: |  |  | 2023-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The security of IMS Data Channels is missing from TS 33.328. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IMS Data Channel security architecture and procedure are specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS Data Channels will be underspecified as the security architecture and procedures are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex N, Annex N.1, Annex N.3, Annex N.3.1, Annex N.3.2, Annex N.3.Y (new), Annex N.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.002: "Network architecture".

[3]	3GPP TS 23.228: "IP Multimedia (IM) Subsystem".

[4]	3GPP TS 33.203: "3G Security; Access security for IP-based services".

[5]	3GPP TS 33.210: "3G Security; Network domain security; IP network layer security".

[6]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[7]	IETF RFC 1035: "DOMAIN NAMES - IMPLEMENTATION AND SPECIFICATION".

[8]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[9]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)".

[10]	IETF RFC 3550:  "RTP: A Transport Protocol for Real-Time Applications".

[11]	IETF RFC 3830: "MIKEY: Multimedia Internet KEYing".

[12]	IETF RFC 4567: "Key Management Extensions for Session Description Protocol (SDP) and Real Time Streaming Protocol (RTSP)".

[13]	IETF RFC 4568: "Session Description Protocol (SDP) Security Descriptions for Media Streams".

[14]	IETF RFC 6043: "MIKEY-TICKET: Ticket-Based Modes of Key Distribution in Multimedia Internet KEYing (MIKEY)".

[15]	IETF RFC 4771: "Integrity Transform Carrying Roll-Over Counter for the Secure Real-time Transport Protocol (SRTP)".

[16]	Otway, D. and Rees, O. 1987: "Efficient and timely mutual authentication." SIGOPS Oper. Syst. Rev. 21, 1 (Jan. 1987), 8-10.

[17]	Void

[18]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP)".

[19]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[20]	3GPP TS 29.162: "Interworking between the IM CN subsystem and IP networks ".

[21]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[22]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[23]	Void

[24]	IETF RFC 6714: "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)”.

[25]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM), Core Network (CN) subsystem".

[26]	IETF RFC 4575: "A Session Initiation Protocol (SIP) Event Package for Conference State".

[27]	GSM Association, Rich Communication Suite 5.1 Advanced Communications Services and Client Specification, Version 1.0, August 2012.

[28]	3GPP TS 24.247: "Messaging service using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[29]	IETF RFC 5365: "Multiple-Recipient MESSAGE Requests in the Session Initiation Protocol (SIP)".

[30]	Void

[31]	IETF RFC 5652: "Cryptographic Message Syntax (CMS)".

[32]	IETF RFC 5083: " Cryptographic Message Syntax (CMS) Authenticated-Enveloped-Data Content Type".

[33]	IETF RFC 3565: "Use of the Advanced Encryption Standard (AES) Encryption Algorithm in Cryptographic Message Syntax (CMS)".

[34]	ITU-T recommendation T.38 (09/2010): "Procedures for real-time Group 3 facsimile communication over IP networks".

[35]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[36]	IETF RFC 6347: "Datagram Transport Layer Security Version 1.2".

[37]	IETF RFC 7325:"UDP Transport Layer (UDPTL) over Datagram Transport Layer Security (DTLS)".

[38]	Void

[39]	IETF RFC 8826: "Security Considerations for WebRTC".

[40]	IETF RFC 5763: "Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS)".

[41]	IETF RFC 5764: "Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)".

[42]	IETF RFC 8832: "WebRTC Data Channel Establishment Protocol".

[43]	IETF RFC 8851: "Secure/Multipurpose Internet Mail Extensions (S/MIME) Version 4.0 Message Specification".

[44]	IETF RFC 8855: "The Binary Floor Control Protocol (BFCP)".

[45]	IETF RFC 8866: "SDP: Session Description Protocol".

[46]	IETF RFC 7714: "AES-GCM Authenticated Encryption in the Secure Real-time Transport Protocol (SRTP)".

[XX]	IETF RFC 8841: "Session Description Protocol (SDP) Offer/Answer Procedures for Stream Control Transmission Protocol (SCTP) over Datagram Transport Layer Security (DTLS) Transport".

[SS]	IETF RFC 8831: "WebRTC Data Channels".

[DD]	IETF RFC 8864 : "Negotiation Data Channels Using the Session Description Protocol (SDP)".

*** 2nd CHANGE ***

Annex N (normative): 
IMS media plane security interworking for WebRTC access to IMS and IMS data channels

# N.1	General

This annex describes the additional IMS media plane security features that are necessary to support WebRTC IMS Clients access to IMS as well as IMS data channels

# N.2	Media security for RTP

## N.2.1	General

According to IETF RFC 8826  [39], all RTP traffic generated or received by a WebRTC client shall be protected with SRTP, using DTLS-SRTP [40, 41] as the key management protocol. This means that if a WebRTC IMS Client is supposed to be able to communicate with existing IMS endpoints (e.g. IMS UE or PSTN GW), DTLS-SRTP and SRTP shall be terminated at an intermediate node.

This clause describes the additional procedures and interface extensions required to support end-to-access-edge (e2ae) security for RTP using DTLS-SRTP and SRTP.

## N.2.2	e2ae security for RTP using DTLS-SRTP

E2ae protection of RTP using DTLS-SRTP is similar to e2ae protection of MSRP using TLS/TCP and the session establishment procedures are therefore largely the same. In both cases certificate fingerprints need to be exchanged over SDP and the media has to be anchored in IMS by inserting a gateway on the media path. Similarly as for e2ae protection using SDES and TLS, the signalling path between the WebRTC IMS Client and the eP-CSCF needs to be secured.

Figure N.2.2-1 shows the originating procedure for e2ae protection of RTP using DTLS-SRTP. The terminating procedure is similar and is not shown here. Note that no assumption is made on the interface between the WebRTC IMS client and the eP-CSCF except that it is SDP based and integrity protected.

Since only e2ae security is supported at the moment, the WebRTC IMS Client is required to include the indication "e2ae-security requested by UE" in every offer it creates.

It is assumed that the eP-CSCF is aware of the fact the IMS UE is a WebRTC IMS Client and automatically applies e2ae security for terminating calls.  Therefore, unlike the existing e2ae security for RTP and MSRP, there is no need for the IMS UE to explicitly indicate support of e2ae security during registration.

NOTE: Void

The DTLS-SRTP profile to use is described in Annex O of this document.

Figure N.2.2-1: E2ae protection of RTP based on DTLS-SRTP

# N.3	Media security for WebRTC and IMS data channels

## N.3.1	General

This clause describes how end-to-access-edge (e2ae) and end-to-end (e2e) security is achieved for WebRTC Data Channels (see IETF RFC 8831 [SS], IETF RFC 8841 [XX], and IETF RFC 8864 [DD]). In addition the end-to-DC-edge (e2DCe) security is specified. The end-to-DC-edge (e2DCe) is defined to be the path between the UE and DCMF.

WebRTC-compatible browsers use SCTP over DTLS as transport protocol for peer-to-peer data. A WebRTC Data Channel is defined as two unidirectional SCTP streams, one in each direction, which are managed together as a single entity (see IETF RFC 8831 [SS]).  The application protocol which runs on top of the WebRTC Data Channel is not specified and the JavaScript is free to implement any protocol it requires.

The application protocols that a WebRTC IMS Client may need to support are MSRP, BFCP, T.140, and T.38. A DCMTSI IMS Client [35] also supports the unspecified application protocol approach used by WebRTC Data Channel with, for example, a related web page and JavaScript handling the needed transmission and protocol format actions.

Figure N.3.1-1 shows the common protocol stack and the required protocol translation for a WebRTC IMS Client, where the WebRTC Data Channel stack is not used in IMS core network or towards the peer. The transport protocol that the IMS-AGW applies on the remote side (marked X in the figure) depends on the application protocol. For MSRP and BFCP X=TCP, for T.140 X=RTP/UDP, and for T.38 X=UDPTL/UDP. In general, the IMS-AGW will forward the application protocol messages transparently. The only exception is MSRP messages which contain IP address information and therefore needs to be re-written by the IMS-AGW. This can however be avoided if both endpoints support the MSRP CEMA extension [24].

T.140 (real-time text) and T.38 (fax) are included here for sake of completeness. These are legacy protocols and are not expected to be commonly used.

Editor’s Note: The final list of supported application protocols (e.g., MSRP, BFCP, T.140, and T.38) is to be decided by CT groups.

Figure N.3.1-1: Protocol stack for WebRTC IMS Client (WIC) Data Channels

Figures N.3.1-2 and N.3.1-3 are based on TS 23.228 [3] and show two examples of the protocol stack used by DCMTSI [35] Clients. Figure N3.1-2 shows a protocol stack for the HTTP proxy configuration mode for the Bootstrap Data Channel or Application Data Channel in case DCMF is anchoring DTLS.

Figure N.3.1-2: Protocol stack for IMS bootstrap data channels (TS 23.228[3])

Figure N.3.1-3 shows protocol stack for the UDP proxy configuration mode for the Application Data Channel case, providing a Person2Application/Application2Person/Person2Person Data Channel Application.

Figure N.3.1-3: Protocol stack for P2P/P2A/A2P IMS application data channels without DCMF (TS 23.228[3])

## N.3.2	e2ae security for WebRTC data channels

E2ae security for WebRTC Data Channels for a WebRTC IMS Client (WIC) is achieved in the same way as e2ae security for MSRP over TLS/TCP.  In both cases certificate fingerprints need to be exchanged over SDP and the media has to be anchored in IMS by inserting a gateway on the media path. To ensure the integrity of the certificate fingerprint the signalling path is assumed to be protected.

Figure N.3.2-1 shows the originating procedure for e2ae protection of WebRTC Data Channels. The terminating procedure is similar and is not shown here. Note that no assumptions are made on the interface between the WebRTC IMS Client and eP-CSCF except that it is SDP-based and integrity protected.

Since only e2ae security is supported at the moment for WebRTC IMS Client (WIC), the WebRTC IMS Client is required to include the indication "e2ae-security requested by UE" in every offer it creates.

It is assumed that the eP-CSCF is aware of the fact the IMS UE is a WebRTC IMS Client and automatically applies e2ae security for terminating calls.  The P-CSCF is aware of a DCMTSI client, based on the IMS data channel media feature tag used by the DCMTSI client during registration. Therefore, unlike the existing e2ae security for MSRP over TLS/TCP, there is no need for the IMS UE to indicate support of e2ae security during registration.

Figure N.3.2-1: E2ae protection of WebRTC Data Channels

NOTE 1:	The method for negotiating the application protocol (e.g. MSRP) and configuring the WebRTC Data Channel (e.g. setting stream identifiers, choosing between reliable or unrealiable transmission, etc) is defined in the corresponding stage 3 specification, e.g., based on IETF RFC 8832 [42] or IETF RFC 8864 [DD].

NOTE 2:	From a security perspective, it is safe to multiplex several WebRTC Data Channels (e.g. one for MSRP and one for BFCP) on top of a single SCTP association and DTLS connection.  However, there may be other, non-security related reasons that prevent this option.

NOTE 3:	IMS Data Channel (DCMTSI) clients use a=dcmap lines from IETF RFC 8864 [DD] in SDP, in addition to the SDP lines indicated above, while WebRTC clients do not. See also Figure N.3.X-1.

## N.3.Y	e2DCe security for IMS data channels

For DCMTSI clients using IMS data channel media, e2DCe security is achieved by anchoring DTLS in Data Channel Media Function (DCMF) while IMS Access GW is anchoring UDP but transparently letting through layers above UDP, to avoid unnecessary DTLS decryption/encryption operations, but otherwise handling certificates and fingerprints as above. See Figure N.3.1-2.

For IMS data channel (DCMTSI) clients, both e2DCe and e2e security shall be supported for IMS data channels and e2DCe security support may be included in SDP offers and answers.

## N.3.X	e2e security for IMS data channels

E2e security for IMS Data Channels for DCMTSI clients is achieved in a similar way as for e2DCe security. See clause N.3.Y. In both cases, certificate fingerprints need to be exchanged over SDP between the peers and certificates are exchanged through DTLS handshake in-band on the media path. The media may be anchored in IMS by inserting a gateway on the media path. To ensure the integrity of the certificate fingerprint, the signalling path is assumed to be protected.

Figure N.3.X-1: E2E protection of IMS Data Channels

*** END OF CHANGES ***