3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	S3-234297

# Berlin, Germany, 22-26 May 2023														    was S3-234129

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | draft | rev | - | Current version: | 17.1 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR editorial updates on resource owner aware northbound access to APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPPY |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The pCR provides editorial updates for security for resource owner aware northbound access to APIs |  |  |  |  |  |  |  |  |
| Summary of change: |  | The editor's note in 5.2 can be deleted, as there was no update of the RNAA figure in 23.222. Editorial in 5.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved editor's note |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

++++++ First Change ++++++

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AEF	API Exposing Function

API	Application Programming Interface

CAPIF	Common API Framework

JSON	JavaScript Object Notation

JWT	JSON Web Token

KDF	Key Derivation Function

PKI	Public Key Infrastructure

PSK	Pre-Shared Key

RNAA	Resource owner-aware northbound API access

TLS	Transport Layer Security

++++++ Next Change ++++++

# 5	Functional security model

## 5.1	General functional security model

Figure 5.1-1 shows the functional security model for the CAPIF architecture. The interfaces CAPIF-1, CAPIF-1e, CAPIF-2, CAPIF-2e, CAPIF-3, CAPIF-4, CAPIF-5, CAPIF-3e, CAPIF-4e, CAPIF-5e, CAPIF-7 and CAPIF-7e are defined in 3GPP TS 23.222 [3] and support the CAPIF functionality defined in 3GPP TS 23.222 [3]. CAPIF-1, CAPIF-2, CAPIF-3, CAPIF-4, CAPIF-5 and CAPIF-7 are interfaces that lie within the PLMN trust domain while the CAPIF-1e, CAPIF-2e, CAPIF-3e, CAPIF-4e, CAPIF-5e and CAPIF-7e interfaces are CAPIF core and AEF access points for API Invokers outside of the PLMN trust domain.

Security for the CAPIF-1, CAPIF-2, CAPIF-3, CAPIF-4, CAPIF-5 and CAPIF-7 interfaces support TLS and are defined in subclauses 6.2, 6.4 and 6.6 of the present document. Security for the CAPIF-1e, CAPIF-2e and CAPIF-7e interfaces support TLS, and are defined in subclause 6.3, subclause 6.5, and subclause 6.9 respectively.

Security for the CAPIF-3e, CAPIF-4e and CAPIF-5e interfaces support NDS/IP security to secure communication between different IP security domains. This avoids multiple secure connections between API provider domain and CAPIF core domain by leveraging the NDS/IP security procedures specified in TS 33.210 [2].

Authentication and authorization are required for both API invokers that lie within the PLMN trust domain and API invokers that lie outside of the PLMN trust domain. For an API invoker that is outside of the PLMN trust domain, the CAPIF core function in coordination with the API exposing function utilizes the CAPIF-1e, CAPIF-2e and the CAPIF-3 interfaces to onboard, authenticate and authorize the API invoker prior to granting access to CAPIF services. Security flow diagrams for onboarding security, CAPIF-1e security and CAPIF-2e security can be found in Annex B. When the API invoker is within the PLMN trust domain, the CAPIF core function in coordination with the API exposing function perform authentication and authorization of the API invoker via the CAPIF-1, the CAPIF-2 and the CAPIF-3 interfaces prior to granting access to CAPIF services. Authentication and authorization of API invokers (both internal and external to the PLMN trust domain) is specified in clause 6 of the present document.

Figure 5.1-1: CAPIF functional security model

## 5.2	Functional security model supporting RNAA

Figure 5.2-1 shows the functional security architecture of CAPIF when RNAA is supported.  The authorization function itself is a part of the CCF. The Oauth client and the CCF shall communicate using https.

Figure 5.2-1: CAPIF supporting RNAA functional security model

Editor's note: the above figure will need alignment with SA6.

++++++ Next Change ++++++

## 6.5	Security procedures for CAPIF-2e reference point

### 6.5.Y2e	Authentication and authorization for RNAA

#### 6.5.Y2e.1	General

The authorization for RNAA shall use token based authorization using oauth2 framework. The API invoker has the role of the OAuth client. The CCF has the role of the OAuth authorization server. The AEF has the role of the resource server.

The access tokens used for RNAA can contain the resource owner identity claim and other claims. Within the token, GPSI is used as identifier. To avoid privacy issues, GPSI needs to be different from MSISDN, SUPI etc. AEF should do the authorization check to the API invocation request containing the token against the claims including the resource owner identity. As the claim in the token includes resource owner identity, there is no need for additional UE authentication in API invocation. Moreover, the token should be able to restrict the API invoker to a specific resource (e.g., location, QoS, PDN connectivity status). For Oauth flows involving redirection, authentication between CCF/AUF and UE should be performed after API Invoker redirects the UE to CCF/AUF. Based on the operator's policy, mutual authentication between resource owner and CCF/AUF can be performed. The authentication method is up to operator policy (e.g. an already deployed mechanism, or AKMA, or GBA).

In case of an external AF (i.e., not the application on the UE) as API invoker, for mutual authentication of API invoker AF and API exposing function, TS 33.122 [5] is reused.

6.5.Y2e.2	Authorization using oauth … flow

Editor's note: this clause describes authorization making use of the relevant oauth flow. Additional subsubsubclauses may be added for additional oauth flows.

#### 6.5.Y2e.n	Revocation

API Exposing Function needs to be informed about revocation if this is necessary to ensure correct handling of revocation.

Editor's note: this clause describes the revocation procedure, unless this is taken care of by SA6 as it is for non RNAA use cases.

+++++ End of Changes +++++