3GPP TSG-SA3 Meeting #112 	S3-233763

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0166 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of the roaming restriction for Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add a 3GPP method to fresh the KAKMA based on the factor that KAF is about to be expired. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add details on when the KAF is about to be expired, the AF can indicate it to the AAnF, and the AAnF can determine whether to refresh the KAKMA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to use the KAKMA refresh clause is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1;6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes****

4.1	Reference model

Figure 4.1-1 shows a fundamental network model of AKMA, as well as the interfaces between them.

Figure 4.1-1: Fundamental Network Model for AKMA

NOTE:	Figure 4.1-1 shows the case where AAnF is deployed as a standalone function. Deployments can choose to collocate AAnF with AUSF or with NEF according to operators' deployment scenarios.

Figure 4.1-2 shows the AKMA architecture using the reference point representation.

Figure 4.1-2: AKMA Architecture in reference point representation for (a) internal AFs of HPLMN and (b) external AFs

The AKMA service requires a new logical entity, called the AKMA Anchor Function (AAnF).

The AKMA Architecture in Figure 4.1-2 is applicable to both roaming scenario and non-roaming scenario:

-	non-roaming: UE is in HPLMN and accessing an AF;

-	roaming scenario#1: UE is in VPLMN and accessing an internal HPLMN AF;

-	roaming scenario#2: UE is in VPLMN and accessing an internal VPLMN AF;

-	roaming scenario#3: UE is in VPLMN and accessing an external AF in the Data Network.

**** Next Changes****

4.4.0	General

The following security requirements are applicable to AKMA:

-	AKMA shall reuse the same UE subscription and the same credentials used for 5G access.

-	AKMA shall reuse the 5G primary authentication procedure and methods specified in TS 33.501 [2] for the sake of implicit authentication for AKMA services.

-	The SBA interface between the AAnF and the AUSF shall be confidentiality, integrity and replay protected.

-	The SBA interface between AAnF and AF/NEF shall be confidentiality, integrity and replay protected.

-	The AKMA Application Key (KAF) shall be provided with a maximum lifetime.

**** End of Changes****