3GPP TSG-SA3 Meeting #113 	S3-234532

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0187 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AKMA Service disable or withdrawn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, ZTE, ChinaMobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the existing AKMA specs (TS 33535), operators can trigger the AKMA context removal API (Naanf_AKMA_ContextRemove) to request AAnF to remove the active AKMA context. This may happen due to the fact that UE AKMA services are expired in the subscription (operator can sell AKMA service subscriptions on monthly basis) or the AKMA subscription is withdrawn. However, UE and AF are not aware of the AKMA context removal. UE and AF can still continue the session for a long time even if UE's AKMA subscription is withdrawn. As of now, there is no way to stop the Ua* connection when AKMA services are withdrawn. It may lead to a revenue loss for the operator because AF can continue to provide the service to UE even after the AKMA subscription has expired or been withdrawn. Secondly, it is a bad solution from a security point of view because UE and AF continue to use the service even after AMKA subscription is withdrawn. When DNN is withdrawn or Slice is withdrawn, UDM informs SMF, and SMF immediately closes the PDU connection related to the slice/DNN. But there is no way to stop AKMA service after it is withdrawn. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AF subscribes to receive AKMA context removal notifications from AAnF. When AKMA context is removed at AAnF, AAnF notifies AF about the same. AF accordingly takes appropiate actions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Subscriber continue to consume AKMA services for days even after AKMA service is withdrawn |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1, 6.2.1, 7.1.3, 7.15, 7.1.x, 7.3.2, 7.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES

### 6.6.1	General

This procedure is used to remove the AKMA context in the AAnF. NF consumers may initiate this procedure due to local policy.

Figure 6.6.1-1: AAnF AKMA context removal procedure

1. NF initiates an AAnF AKMA context removal procedure to delete the AKMA context in AAnF.

2. NF discovers the AAnF of the UE, as specified in clause 6.7 and sends a Naanf_AKMA_Context_Remove request to AAnF to remove AKMA context for the UE.

3. AAnF shall delete AKMA Context (e.g. SUPI, A-KID and KAKMA) from its local database.

4. AAnF sends a Naanf_AKMA_Context_Remove response to NF.

5. AAnF determines if AF(s) have subscribed to receive notifications for AKMA context removal; if yes, steps 6 and 7 are executed. Otherwise, steps 6 and 7 are skipped.

6. If AF(s) are determined at step 5, the AAnF shall send notifications to the subscribed AF(s) about AKMA context removal via Naanf_AKMA_ContextRemNotification.

7. The AF shall send the response.

************ NEXT CHANGES**********

### 6.2.1	AAnF response with UE Identity

Figure 6.2-1 shows the procedure used by the AF to request application function specific AKMA keys from the AAnF, when the AF is located inside the operator's network.

Figure 6.2-1: KAF generation from KAKMA

Before communication between the UE and the AKMA AF can start, the UE and the AKMA AF need to know whether to use AKMA. This knowledge is implicit to the specific application on the UE and the AKMA AF or indicated by the AKMA AF to the UE (see clause 6.5).

1.	The UE shall generate the AKMA Anchor Key (KAKMA) and the A-KID from the KAUSF before initiating communication with an AKMA Application Function. When the UE initiates communication with the AKMA AF, it shall include the derived A-KID (see clause 6.1) in the Application Session Establishment Request message. The UE may derive KAF before sending the message or afterwards.

2.	If the AF does not have an active context associated with the A-KID, then the AF selects the AAnF as defined in clause 6.7, and sends a Naanf_AKMA_ApplicationKey_Get request to AAnF with the A-KID to request the KAF for the UE. The AF also includes its identity (AF_ID) in the request. If AF wants to receive a notification for AKMA context removal, the AF shall include context removal URI in the Naanf_AKMA_ApplicationKey_Get request. Based on the context removal URI, the AAnF shall create an implicit subscription for the AF for the AANF to later notify the AF about AKMA context removal as defined in 6.6.1. Implicit subscription has an expiration time set by operator policy.

AF_ID consists of the FQDN of the AF and the Ua* security protocol identifier (see Annex A.4). The latter parameter identifies the security protocol that the AF will use with the UE.

The AAnF shall check whether the AAnF can provide the service to the AF based on the configured local policy or based on the authorization information available in the signalling (i.e., Oauth2.0 token). If it succeeds, the following procedures are executed. Otherwise, the AAnF shall reject the procedure.

The AAnF shall verify whether the subscriber is authorized to use AKMA based on the presence of the UE specific KAKMA key identified by the A-KID.

If KAKMA is present in AAnF, the AAnF shall continue with step 3.

If KAKMA is not present in the AAnF, the AAnF shall continue with step 4 with an error response.

3.	Once receving the request from the AF, if the AAnF determines this specific AF needs GPSI, according to its local policy, the AAnF sends a Nudm_SDM_Get Request to the UDM to fetch the GPSI of the UE. If the specific AF does not need GPSI, the AAnF shall continue with step 5.

4.	The UDM responds with the GPSI of the UE. The AAnF shall store the received GPSI as part of UE’s AKMA context.

5.	The AAnF derives the AKMA Application Key (KAF) from KAKMA if it does not already have KAF.

The key derivation of KAF shall be performed as specified in Annex A.4.

6.	The AAnF sends Naanf_AKMA_ApplicationKey_Get response to the AF with SUPI, KAF and the KAF expiration time. Whether to send SUPI or GPSI is determined by AAnF based on the local policy.

7.	The AF sends the Application Session Establishment Response to the UE. If the information in step 4 indicates failure of AKMA key request, the AF shall reject the Application Session Establishment by including a failure cause. Afterwards, UE may trigger a new Application Session Establishment request with the latest A-KID to the AKMA AF.

************ NEXT CHANGES**********

### 7.1.3	Naanf_AKMA_ApplicationKey_Get service operation

Service operation name: Naanf_AKMA_ApplicationKey_Get.

Description: The NF consumer requests AKMA Application Key and UE ID from the AAnF.

Input, Required: A-KID, AF_ID

Input, Optional: context removal URI.

Output, Required: KAF, KAF expiration time and SUPI or GPSI.

Output, Optional: None.

************ NEXT CHANGES**********

### 7.1.5	Naanf_AKMA_ApplicationKey_ AnonUser_Getservice operation

Service operation name: Naanf_AKMA_ApplicationKey_AnonUser_Get.

Description: The NF consumer requests only the AKMA Application Key from the AAnF. This service is for allowing anonymous user access to the AF based on A-KID (i.e., UE identification is not required at the AF). The A-KID functions as a temporary user identifier.

Input, Required: A-KID, AF_ID

Input, Optional: context removal URI.

Output, Required: KAF, KAF expiration time.

Output, Optional: None.

************ NEXT CHANGES**********

### 7.1.x	Naanf_AKMA_ContextRemNotification service operation

Service operation name: Naanf_AKMA_ContextRemNotification

Description: AAnF notifies the NF consumer about AKMA context is removed.

NOTE:	The AF is implicitly subscribed to receive Naanf_AKMA_ContextRemNotification service operation.

Input, Required: A-KID

Input, Optional: None

Output, Required: None

Output, Optional: None

************ NEXT CHANGES**********

### 7.3.2	Nnef_AKMA_ApplicationKey_Get service operation

Service operation name: Nnef_AKMA_ApplicationKey_Get.

Description: The NF consumer requests the NEF to provide AF related key material.

Input, Required: A-KID, AF_ID

Input, Optional: UEID not needed indication, context removal URI.

Output, Required: KAF, KAF expiration time.

Output, Optional: GPSI (external ID).

************ NEXT CHANGES**********

### 7.3.x	Nnef_AKMA_ContextRemNotification service operation

Service operation name: Nnef_AKMA_ContextRemNotification

Description: NEF notifies the NF consumer about AKMA context is removed.

Input, Required: A-KID

Input, Optional: None

Output, Required: None

Output, Optional: None

************ END OF CHANGES**********