3GPP TSG-SA3 Meeting #113 	S3-234583

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0003 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the issue when SLPTK ID is about to wrap around |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to 6.4.4, SLPTK ID is a counter in the UE set to a unique value that has not been previously used together with the same SLPGK and associated SLPGK ID. Every time a new SLPTK needs to be derived, the SLPTK ID counter is incremented. Therefore, if the STPTK ID is wrap around, the SLPTK ID will be used together with the same SLPGK if the key is not changed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the issue when SLPTK ID is about to wrap around |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SLPTK ID will be used together with the same SLPGK if the key is not changed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 33.220 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st CHANGE ***

6.4.4.2	Security flows for broadcast/groupcast communication

Figure 6.4.4.2-1: Security flows for broadcast/groupcast communication

0a and 0b. Both sending and receiving UEs shall be provisioned with the parameters/policy for Ranging/SL positioning service as specified in clause 5.1 of TS 23.586 [2].

1a.	The sending UE shall establish a secure connection with the Sidelink Positioning Key Management Function (SLPKMF) based on the security procedures specified in clause 5.2.5 of TS 33.503 [6]. The sending UE sends a Key Request message to Sidelink Positioning Key Management Function (SLPKMF) including the Ranging/SL positioning application identifier provisioned in step 0a, and UE security capabilities.

1b.	The SLPKMF shall reply with the Key Response message containing the Sidelink Positioning Group Key (SLPGK), the Sidelink Positioning Group Key ID (SLPGK ID), the validity time, and the chosen ciphering and integrity algorithms. The SLPKMF may be locally configured with the UE's authorization information. Otherwise, the SLPKMF interacts with the UDM of the UE to retrieve the UE's authorization information. The chosen ciphering and integrity algorithms are determined by SLPKMF based on the received UE security capabilities in step 1a. The Key Response message may include multiple SLPGK and SLPGK ID pairs with different validity times. Group member ID shall be assigned by the SLPKMF which is included in the Key Response message. As an alternative, the sending UE may generate a Group member ID randomly.

NOTE 1: 	For V2X capable UEs, the security materials (e.g. SLPGK, SLPGK ID, validity time) and the ciphering and integrity algorithms used for broadcast/groupcast communication are provisioned at the application, which is out of the scope of the present document.

NOTE 2: 	In case the SLPLMF of a receiving UE is different from the SLPKMF of a sending UE, the provisioning of security materials as specified in clause 6.1.3.2 in TS 33.503 [6] is reused.

NOTE 3: 	Sidelink Positioning Group refers to a specific Ranging/SL positioning service. Accordingly, Group member ID refers to the identifier of the UE that is authorized to use the Ranging/SL positioning service.

2.	The receiving UE shall perform a Key Request procedure to get security materials from the SLPKMF as described in step 1. This may happen any time before step 5.

3.	The sending UE shall derive the Sidelink Positioning Traffic Key (SLPTK) from SLPGK using Group member ID, and SLPTK ID as specified in Annex A.3 of present document. SLPTK ID is a counter set to a unique value in the sending UE that has not been previously used together with the same SLPGK and the associated SLPGK ID. The UE shall require a new SLPGK as described in step 1 when the SLPTK ID is about to wrap around. The UE shall calculate the Sidelink Positioning Encryption Key (SLPEK) and Sidelink Positioning Integrity Key (SLPIK) from SLPTK using the chosen ciphering and integrity algorithms, respectively as specified in Annex A.4 of present document.

4.	The sending UE shall protect the message as described in clause 6.4.4.3.1 and send the message.

5.	Upon receiving the message matching the SLPGK ID, the receiving UE shall calculate SLPTK, SLPEK and SLPIK if it has not calculated them. The receiving UE derives security keys as in step 3 using the SLPGK ID, SLPTK ID and Group member ID (if it is included) in the received message. Then, the UE shall decrypt the message and verifies the integrity of the message as described in clause 6.4.4.3.2.

*** END OF CHANGES***