3GPP TSG-SA3 Meeting #113 	S3-234627

# Chicago, US, 6 - 10 November 2023											            revision of S3-23yyyy

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1823 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security for NSWO support in SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| 30 |  |  |  |  |  |  |  |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on discussion in S3-234626, there is the following observations: Observation 1: EAP-AKA’ introduces new CK’ and IK’ derivation function binding the access network name. During the primary authentication using EAP-AKA’, the access network name is binding to the current serving PLMN ID. However, in case of NSWO SNPN case, the access network name remains a static string. Observation 2: In case of SNPN NSWO, if a malicious WLAN AN of SNPN1 broadcasts a different SNPN ID (i.e. SNPN2), UEs that prefer SNPN2 will be spoofed to connect to the malicious WLAN AN of SNPN1. This is because NSWO authentication does not bind with the current serving network ID, leading to severe security threats in the user plane data, such as eavesdropping, impersonation, tampering, and so on. It is proposed that similar like current 4G or 5G primary authentication, both UE and AUSF uses current serving network ID for CK’, IK’ derivation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed the following: UE and NSWOF needs to set the access network identity to “5G:NSWO” and SN ID (i.e. serving SNPN ID) for CK’, IK’ derivation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Primary authentication for SNPN NSWO is unsecure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | I.10.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

I.10.5.2	NSWO support in SNPN without CH

5G NSWO procedures are defined in Annex S.3.2. For SNPN the procedures are extended to usage of any key-generating EAP-method as follows:

Steps 1-2 are performed as described in Annex S.3.2.

In step 3, the SUCI can be of type anonymous SUCI if the construction of SUCI as described in clause 6.12 cannot be used and if the employed EAP method supports SUPI privacy.

Steps 4 is performed as described in Annex S.3.2.

Step 5 is performed as described in Annex S.3.2 with difference that the NSWOF shall set the Access Network Identity to "5G:NSWO" and SN ID. The Access Network Identity is the concatenation of a service code (i.e. "5G:NSWO") and the SN ID with a separation character ":" such that the service code prepends the SN ID.

Steps 6 is performed as described in Annex S.3.2.

7. Upon reception of the Nudm_UEAuthentication_Get Request, the UDM invokes SIDF to de-conceal SUCI to gain SUPI if the received SUCI is not an anonymous SUCI. For selection of authentication methods, the statements in Annex I.2.2.1 apply. In case of SNPN, the UDM selects authentication method based on the NSWO indicator, subscription data and/or local configuration. The authentication method may include EAP-AKA’ or any other key-generating EAP authentication method. The UDM returns the selected authentication method to the AUSF.

8. Authentication is performed between the AUSF and UE using the selected EAP method. After a successful authentication the AUSF derives the MSK key and does not generate the KAUSF, as indicated by the NSWO indicator and as described for the PLMN case in Annex S.3.2. In additionally ,UE shall construct Access Network Identity using the same way as described in step 5.

Steps 9-11 are performed as described in steps 16-18 of Annex S.3.2.

*** END CHANGES ***