3GPP TSG-SA3 Meeting #113 	S3-234737

# Chicago, US, 6 - 10 november 2023													(revision of S3-yyxxxx)

# Source:	Ericsson

# Title:	Security for Subsequent Conditional PSCell Addition or Change

# Document for:	Approval

Agenda Item:	4.1.15

# 1	Decision/action requested

This proposal provides an update to S3-234164.

# 2	References

[1]	S3-234164 "Security for Selective SCG Activation"

[2]	R2-2311607 "Reply LS on security for selective SCG activation"

# 3	Rationale

This contribution is based on the following rationale:

- RAN2 decided that UE can send uplink traffic to SN before the SN Reconfiguration Complete message from the MN reaches the SN. Therefore, all security information needs to be in place in the SN before that, which rules out Option 2.

The above is based on that the RAN2 LS R2-2311607. RAN2 has indicated the following in the Additional NOTE: "In current PSCell change and CPAC scenarios, SN is provided with the security keys upon configuration of SCPAC as UL data transmission may start in parallel with the transmission of the RRC Reconfiguration Complete message. "

- The difference between Option 1 and Option 3 is that in Option 3, the UE does not signal the SN Counter value it used to the MN in the RRC Reconfiguration Complete. Because RAN2 has decided that the UE shall send the counter value in this message, Option 3 can be ruled out.

The above is based on the RAN2 LS R2-2311607. RAN2 has indicated the following in bullet 2: “UE include the selected SK-counter value in the MN RRC Reconfiguration Complete message when UE selects new SK-counter value as part of SCPAC execution”.

Therefore, it is proposed that Option 1 is specified in more detail as of this pCR.

- Specifying a new FC value for the key derivation removes unknown interactions with the keys derived for the regular SCG addition procedure and is the safest approach. The cost of a single FC value is not high.

- Editorial: Despite the name of the LSes on this topic, the name of the feature is Subsequent CPAC. Therefore, references to “Activation” and “Selective SCG” are removed to avoid confusion with other clauses in the specification.

# 4	Detailed proposal

See below proposed updates to S3-234164.

### *** FIRST CHANGE ***

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5G-AN	5G Access Network

5G-RG	5G Residential Gateway

NG-RAN	5G Radio Access Network

5G AV	5G Authentication Vector

5G HE AV	5G Home Environment Authentication Vector

5G NSWO	5G Non-Seamless WLAN Offload

5G SE AV	5G Serving Environment Authentication Vector

ABBA	Anti-Bidding down Between Architectures

AEAD	Authenticated Encryption with Associated Data

AES	Advanced Encryption Standard

AKA	Authentication and Key Agreement

AMF	Access and Mobility Management Function

AMF	Authentication Management Field

NOTE:	If necessary, the full word is spelled out to disambiguate the abbreviation.

ARPF	Authentication credential Repository and Processing Function

AUN3	Authenticable Non-3GPP devices

AUSF	Authentication Server Function

AUTN	AUthentication TokeN

AV	Authentication Vector

AV'	transformed Authentication Vector

BAP	Backhaul Adaptation Protocol

BH	Backhaul

CCA	Client Credentials Assertion

Cell-ID	Cell Identity as used in TS 38.331 [22]

CH	Credentials Holder

CHO	Conditional Handover

CIoT	Cellular Internet of Things

cIPX	consumer's IPX

CKSRVCC	Cipher Key for Single Radio Voice Continuity

cNRF	consumer's NRF

CP	Control Plane

CPAC	Conditional PSCell Addition or Change

cPLMN	consumer's PLMN

cSEPP	consumer's SEPP

CTR	Counter (mode)

CU	Central Unit

DCS	Default Credentials Server

DN	Data Network

DNN	Data Network Name

DU	Distributed Unit

EAP	Extensible Authentication Protocol

EDT	Early Data Transmission

EMSK	Extended Master Session Key

EN-DC	E-UTRA-NR Dual Connectivity

ENSI	External Network Slice Information

EPS	Evolved Packet System

FN-RG	Fixed Network RG

gNB	NR Node B

GUTI	Globally Unique Temporary UE Identity

HRES	Hash RESponse

HXRES	Hash eXpected RESponse

IAB	Integrated Access and Backhaul

IKE	Internet Key Exchange

IKSRVCC	Integrity Key for Single Radio Voice Continuity

IPUPS	Inter-PLMN UP Security

IPX	IP exchange service

KSI	Key Set Identifier

KSISRVCC	Key Set Identifier for Single Radio Voice Continuity

LI	Lawful Intercept

MBSF	Multicast/Broadcast Service Function

MBSSF	Multicast/Broadcast Service Security Function

MBSTF	Multicast/Broadcast Service Transport Function

MeNB	Master eNB

MN	Master Node

MO-EDT	Mobile Originated Early Data Transmission

MT-EDT	Mobile Terminated Early Data Transmission

MR-DC	Multi-Radio Dual Connectivity

MSK	Master Session Key

N3IWF	Non-3GPP access InterWorking Function

NAI	Network Access Identifier

NAS	Non Access Stratum

NDS	Network Domain Security

NEA	Encryption Algorithm for 5G

NF	Network Function

NG	Next Generation

ng-eNB	Next Generation Evolved Node-B

ngKSI	Key Set Identifier in 5G

N5CW	Non-5G-Capable over WLAN

N5GC	Non-5G-Capable

NIA	Integrity Algorithm for 5G

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NSSAI	Network Slice Selection Assistance Information

NSSAA	Network Slice Specific Authentication and Authorization

NSWO	Non-Seamless WLAN Offload

NSWOF	Non-Seamless WLAN Offload Function

PDN	Packet Data Network

PEI	Permanent Equipment Identifier

pIPX	producer's IPX

pNRF	producer's NRF

pPLMN	producer's PLMN

PRINS	PRotocol for N32 INterconnect Security

pSEPP	producer's SEPP

PUR	Preconfigured Uplink Resource

QoS	Quality of Service

RES	RESponse

SCG	Secondary Cell Group

SEAF	SEcurity Anchor Function

SCP	Service Communication Proxy

NOTE: Void.	Security Gateway

SEPP	Security Edge Protection Proxy

SCPAC	Subsequent Conditional PSCell Addition or Change

SgNB	Secondary gNB

SIDF	Subscription Identifier De-concealing Function

SMC	Security Mode Command

SMF	Session Management Function

SN	Secondary Node

SN Id	Serving Network Identifier

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TLS	Transport Layer Security

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

TSC	Time Sensitive Communication

UE	User Equipment

UEA	UMTS Encryption Algorithm

UDM	Unified Data Management

UDR	Unified Data Repository

UIA	UMTS Integrity Algorithm

ULR	Update Location Request

UP	User Plane

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

USIM	Universal Subscriber Identity Module

XRES	eXpected RESponse

### *** NEXT CHANGE ***

### 6.10.2.X	Security mechanism and procedures for Subsequent CPAC

### 6.10.2.X.1		General

In the scenario of Subsequent CPAC activation, the MN may provide one or several candidate CPAC configuration(s) for one or multiple candidate SN(s) to the UE. The UE may select and execute precisely one of these conditional reconfigurations in order to change PSCell based on the measurement results on candidate target PSCells. The conditional reconfiguration for the selected PScell remains valid after the UE selected the target and executed the target cell access procedure. Thus the UE is able to connect to the same SN several times without any further reconfiguration by the network.

### 6.10.2.X.2		Security context initialisation for Subsequent CPAC

In order to prevent key-stream reuse when the UE switches back and forth to the same PSCell or SN, the MN shall generatea sequence of distinct SN Counter values for each SN during the CPAC  activation procedure. Because each SN Counter value is unique w.r.t. the currently active KgNB, the sequences are non-overlapping. These SN Counter valuesequences shall be provided to the UE by the MN. The UE shall store these values.

The MN shall derive the KSNs corresponding to the SN Counter value sequences for each SN from the current KgNB using the inputsgiven in Annex A.x. The MN shall then send the KSN key sequencess and the corresponding SN Counter values for each SN to that SN. The SN shall store the KSN keys and the corresponding SN Counter values in its security context.

Editor’s Note: Further collaborate with RAN2 to decide the specific message for the MN to signal the SN Counter values per SN to UE, and to signal KSN key(s) to SN, respectively.

When the UE accesses a CPAC of a target SN, either for the first time or for a following time based on the same conditional reconfiguration, the UE shall select the first unused SN Counter value from the SN Counter value sequence associated with the target SN. Because all values are distinct, selecting the first unused one ensures that it is not previously used together with the current KgNB. The UE shall then derive the corresponding KSN using the SN Counter value as described in Annex A.x. The UE and the SN shall derive the user plane encryption key, and userplane integrity key when configured, from the KSN for protecting their communications.

The UE shall inform the MN which SN Counter value it used for KSN derivation when accessing the SCG of the target SN. The MN shall signal the received SN Counter value to the SN.  When the UE sends UP messages protected by encryption and integrity keys derived from the KSN to the SN, these may reach the SN before the SN has received the SN Counter value. Therefore, the SN may not be able to prevent synch-failure with the UE on the SN Counter value, but the SN may use it to recover from failure.

There are two scenarios:

- In the first scenario, the UE and the SN may need to activate encryption and integrity protection taking an unused KSN key into use, before the SN has received the SN Counter value from the UE via the MN.

In this scenario, the SN chooses the first unused KSN key for that UE to establish the security with the UE.

- In the second scenario, the SN can map the SN Counter value, received from the UE via the MN, to the corresponding KSN key before receiving the first UP packet from the UE, and the MN can then activate encryption and integrity protection with the UE taking the known-to-be-right KSN key into use.

## Figure 6.10.2.X.3-1: Procedure for Subsequent CPAC

### *** NEXT CHANGE ***

# A.16	Derivation of KSN for dual connectivity

This input string is used when the MN and UE derive KSN during dual connectivity. The following input parameters shall be used:

-	FC =0x79,

-	P0 = Value of the SN Counter as a non-negative integer,

-	L0 = length of the SN Counter value (i.e. 0x00 0x02).

The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB.

### *** NEXT CHANGE ***

# A.x	Derivation of KSN for Subsequent CPAC

This input string is used when the MN and UE derive KSN during dual connectivity for Subsequent CPAC. The following input parameters shall be used:

-	FC =0xYY,

-	P0 = Value of the SN Counter as a non-negative integer,

-	L0 = length of the SN Counter value (i.e. 0x00 0x02).

The input key KEY shall be KgNB when the MN is a gNB.

### *** END OF CHANGES ***