3GPP TSG-SA3 Meeting #113 	S3-234765

Chicago, US, 6 - 10 November 2023

# Source:	Huawei, HiSilicon

# Title:	New WID on Authentication result removal

# Document for:	Approval

# Agenda Item:	6.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WID on Authentication result removal

Acronym:

Unique identifier:	xxx

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

TS 33.501 clause 6.1.4 specifies that the authentication result will be stored by the UDM after the successful authentication, and can be used to link authentication confirmation to the subsequent procedures, in order to prevent certain types of fraud, e.g. fraudulent Nudm_UECM_Registration Request for registering the subscriber's serving AMF in UDM while the subscriber is not actually present in the visited network.

However, in the case that the Purge of subscriber data in AMF after the UE deregisters from the network or the SMC failure after AKA in the registration procedure, UDM still has the authentication result. This may cause security issues. Fraudulent UDM service request like fraudulent Nudm_UECM_ Registration service request sent by the AMF may be accepted by the UDM, and the UDM may store the AMF ID as the current AMF serving for the UE, even though the UE is already deregistered from the serving network. Therefore, in the Purge of subscriber data in AMF or NAS SMC failure case describe above, the authentication result shall be removed, considering the authentication result usage specified in TS 33.501 clause 6.1.4. Therefore, some additional work is required in SA3 for specifying the authentication result removal.

4	Objective

The goal of this work item is to specify the necessary security requirements and procedures for the authentication result removal.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.501 | New subclause under 6.1.4 could be added to add the security procedures about authentication result removal | SA#102 (Dec 2023) |  |

6	Work item Rapporteur(s)

Fei Li, Huawei, lifei43@huawei.com

7	Work item leadership

SA3

8	Aspects that involve other WGs

CT4 will need to refer to the updated SA3 specification for newly defined security procedures.

9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon