3GPP TSG-SA3 Meeting #113	S3-234801

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.328 | CR | 0077 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change the "P-CSCF(IMS AS)" to "IMS AS via the P-CSCF" |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_SEC |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The epxression P-CSCF(IMS AS) has different semantics from e.g. the expression P-CSCF(IMS-ALG). The former is used to indicate that messages flow to the IMS AS via the P-CSCF while the latter is used to indicate that the IMS-ALG may be part of the P-CSCF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the "P-CSCF(IMS AS)" to "IMS AS via the P-CSCF" in order to clarify the semantics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4.1, 7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** 1st CHANGE ****

### 6.2.4	Key management mechanisms for e2DCe protection

#### 6.2.4.1	Endpoints for e2DCe protection

For IMS Data Channel traffic terminated by the DCMF/MRF, the DCMF/MRF shall send DTLS-protected SCTP packets to and accept DTLS-protected SCTP packets from the served UE as requested by the IMS AS via the P-CSCF.

For the definition of the Data Channel Media Function/MRF cf. TS 23.228 [3].

**** 2nd CHANGE ****

### 7.2.4	IMS UE originating procedures for e2DCe

Figure 7.2.4-1 shows the originating session set-up procedures for one or more media stream(s) using e2DCe security.

Figure 7.2.4-1: Originating call flow for e2DCe using DCMF/MRF case

The IMS UE A performs an IMS originating session set-up according to 3GPP TS 23.228 [3], with modifications as described in the following. If both IMS UE and network indicated support for e2DCe security for IMS data channel during registration, then the IMS UE shall request e2DCe security for IMS data channel media streams to be established as described in this clause, unless the IMS UE initiates a procedure for e2e security for an IMS data channel media stream.

The originating procedures for establishing IMS data channels with e2e security are described in clause 7.2.5 of this specification. The IMS UE may learn of a preference for e2e security for a particular session or media stream by explicit user action via the user interface or by the security policy implemented on the IMS UE.

The procedure in the above figure for requesting e2DCe security for a media stream is now described step-by-step.

1.	IMS UE A sends an SDP Offer for a media stream containing cryptographic information, together with an indication "e2DCe-security for IMS data channel requested by UE", to the IMS AS via the P-CSCF .

For e2DCe protection of IMS data channel the cryptographic information contained in the SDP Offer consists of the fingerprint of the certificate of IMS UE and the tls-id attribute, in accordance with RFC 8841 [48].

2.	For each media stream that uses transport "UDP/DTLS/SCTP", the P-CSCF (IMS-ALG) checks for the presence of the indication "e2DCe-security for IMS data channel requested by UE".

If the indication is present and the P-CSCF (IMS-ALG) indicated support of e2DCe-security for IMS data channel during registration, the P-CSCF (IMS-ALG) allocates the required resources, includes the IMS Access GW and Data Channel Media Function/MRF in the media path and proceeds as specified in this clause. If the indication is not present for an IMS data channel media stream, the P-CSCF (IMS-ALG) proceeds for this media stream as described in clause 7.2.5 of the present specification.

NOTE 1: 	The inclusion of the IMS Access GW and Data Channel Media Function /MRF in the media path is needed for the purposes of e2DCe security even if it was not needed otherwise.

NOTE 2:	If an indication for e2DCe security for a media stream is present in an SDP offer but the support for e2DCe security for the respective protocol was not successfully established during registration then this is an error case.

3.	The IMS AS sends the SDP offer towards the S-CSCF.

4.	For e2DCe protection of IMS Data Channel, the S-CSCF stores the received fingerprint of the IMS UE A certificate, performs the required procedures according to TS 23.228 [3], and forwards the SDP Offer to the terminating network.

5.	The S-CSCF receives the SDP Answer from the terminating network.

6.	The S-CSCF sends the SDP answer towards the IMS AS

7.	The IMS AS and the Data Channel Media Function/MRF exchange the cryptographic information.

For e2DCe protection of IMS Data Channel the cryptographic information communicated by the IMS AS to the Data Channel Media Function/MRF consists of the fingerprint of the UE's certificate and tls-id in accordance with RFC 8841 [48]. The IMS AS instructs the Data Channel Media Function/MRF to verify during the subsequent DTLS handshake with the IMS UE (see step 10) that the fingerprint of the certificate passed by the IMS UE during this DTLS handshake matches the fingerprint passed by the IMS AS to the Data Channel Media Function/MRF. In turn, the Data Channel Media Function/MRF communicates the fingerprint of the certificate it is going to use for setting up protection for this media stream to the IMS AS.

8.	The S-CSCF forwards the SDP Answer to the P-CSCF (IMS-ALG).

9.	The P-CSCF (IMS-ALG) sends the SDP Answer to the IMS UE A. After receiving this message, IMS UE A completes the media security setup.

NOTE 3:	The IMS UE can deduce that e2DCe security is used from two facts: first, that the P-CSCF (IMS-ALG) indicated its support for e2DCe security during registration, and second, that the IMS UE requested e2DCe-security in the SDP Offer.

10.	In case of IMS data channel, when the full session setup has been completed, the DTLS connection shall be established between the IMS UE and the Data Channel Media Function/MRF. When subsequently media are sent from or to the IMS UE, the UE and Data Channel Media Function/MRF perform the required DTLS specific cryptographic operations on the media.

**** END OF CHANGES ****