3GPP TSG-SA3 Meeting #113 	S3-234880

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0017 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to the Reference Points in Clause 4.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_SEC |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is a Editor’s Note in clause 4.2.2 of TS 33.533 stating that “Whether NL6 can be used to resolve UE identity between application layer ID and SUPI for protecting UE privacy is to be aligned with SA2”. According to TS 23.586, the UE ID resolution is performed between the GMLC and UDR via the NEF. This means that the NL6 reference point is not used for UE ID resolution. Hence the editor’s note can be removed. In addition, as specified in clause 4.2.1.1, the SLPKMF has the similar functionalities as those of 5G PKMF specified in TS 33.503, which supports the reference point Npc10 with the UDM for UE authorization. For SLPKMF, there are the same requirements on the interaction with the UDM for UE authorization during security procedures for discovery, unicast direct communication, and broadcast/groupcast communication. For example: - It is specified in clause 6.3 “The SLPKMF shall retrieve subscription information from the UDM for authorizing the role claimed by the UE”. - It is also specified in clause 6.4.4.2 “the SLPKMF interacts with the UDM of the UE to retrieve the UE's authorization information”. The above requirements require a reference point between the SLPKMF and UDM similar as Npc10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed the Editor’s Note in clause 4.2.2. - Added the reference point Npc10* between the SLPKMF and UDM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s note is not resolved. Reference point required for Ranging/SL positioning security is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

4.2.2	Reference points

In addition to the reference points specified in clause 4.2 of TS 23.586 [2], the following reference points are added or enhanced for supporting Ranging/SL positioning security architecture:

NL6:	The reference point between the UDM and the GMLC. It is used to transport the UE privacy profile to GMLC for authorization of Ranging/SL Positioning service exposure.

PC8*:	The reference point between the UE and the SLPKMF, which relies on 5GC user plane for transport (i.e. an "over IP" reference point). It is used to transport security materials to UEs for Ranging/SL Positioning discovery and communication.

Npc9*:	The reference point between the SLPKMFs of the UEs subscribed in different PLMNs. It is used to transport security materials between SLPKMFs to support Ranging/SL Positioning services in inter-PLMN scenarios.

Npc10*:	The reference point between the UDM and the SLPKMF. It is used to request subscription information related to Ranging/SL Positioning service from the UDM for UE authorization.

*************** End of the Change ****************