3GPP TSG-SA3 Meeting #113 	S3-234901

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0031 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the Ranging/SL Positioning service exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As defined in TS 23.586, Ranging/SL Positioning service can be exposed to authorized SL Positioning Client UE, 5GC NF or AF to obtain the relative position or distance/direction result between two or more UEs, and also can be used by authorized 5GC NF, AF or LCS client to obtain the absolute position of Target UE if the 5GC NF, AF or the LCS client determines that Ranging/SL Positioning is applicable. Currently, authorization of LCS client for Ranging/SL Positioning service exposure is not specified in SA3 specification. To align to SA2 specification, it is proposed to add LCS client in TS 33.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added LCS Client as one of the entities to be authorized for Ranging/SL Positioning service exposure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with SA2 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.3.2, 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

6.3.1	General

According to clause 4.1 of TS 23.586 [2], a UE capable of Ranging/SL Positioning may take different roles in various Ranging/SL Positioning operations. Each of the UEs in a Ranging/SL Positioning service acts in its own authorized role. The UE shall follow the policy/parameters defined in clause 5.1 of TS 23.586 [2] for authorization with the network. TS 23.586 [2] clause 5.6 also specifies that Ranging/SL Positioning service can be exposed to an authorized SL Positioning Client UE, 5GC NF, AF or LCS client to obtain the relative or absolution distance/direction result between two UEs capable of Ranging/SL positioning.

This clause specifies the authorization requirements and procedures for the operations in Ranging/SL positioning services wherever authorization or privacy check is required.

*************** Start of the 2nd Change ****************

6.3.2	Authorization requirements

The 5G system shall support the authorization of the role of the UE (e.g. as a Target UE/SL Reference UE/SL Positioning Server UE/Located UE) in a Ranging/Sidelink Positioning service.

The 5G system shall support authorization of the UE for Ranging/SL positioning communication in unicast mode, broadcast/groupcast mode.

The 5G system shall support authorization of the AF, 5GC NF, LCS Client or SL Positioning Client UE for Ranging/SL Positioning service exposure.

The 5G system shall support privacy protection of the to-be-measured UEs for Ranging/SL Positioning service exposure.

*************** Start of the 3rd Change ****************

6.3.5	Procedures for authorization of AF/5GC NF/LCS Client for Ranging/SL positioning service exposure

For the authorization of the AF, 5GC NF or LCS client for Ranging/SL Positioning service exposure, the SL-MT-LR procedure specified in TS 23.273 [3] is taken as the baseline. The authorization shall be performed towards all the n UEs (n ≥ 2), i.e. UE1, UE2, ..., UEn in the request message. If all of the UEs don't grant permission for Ranging/SL Positioning exposure, the GMLC shall reject the service request from the AF, 5GC NF or LCS client.

When receiving the Ranging/SL Positioning service request from the AF, 5GC NF or LCS client, the GMLC interacts with the UDM to check the UE privacy profile. The UE LCS Privacy Profile defined in clause 5.4.2 of TS 23.273 [3] is taken as the baseline for the UE privacy profile for Ranging/SL positioning services.

NOTE:	The details of the UE privacy profile for Ranging/SL positioning services needs to be align with SA2.

The GMLC interacts with the AMF to request the ranging result of UEs, which may include an indication of a privacy related action. If the indicator of privacy check related action indicates that the UE must either be notified or notified with privacy verification, a notification invoke message is sent to the UE if the signalling connection established. However, if the Ranging/SL Positioning service is disallowed by the UE, or signalling connection establishment fails and UE notification (including UE notification with privacy verification) is required, the AMF shall provide failure response to the GMLC.

*************** End of the Changes ****************