3GPP TSG-SA3 Meeting #113	S3-234955

# Chicago, United States, 6th Nov 2023 - 10th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.514 | CR | 0012 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add UDM SCAS test case for checking the authentication verification of a synchronization failure message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A test case for detecting the processing of an unverified AUTS parameter from synchronization failure messages is not yet specified in the UDM SCAS tests, resulting in a lack of test coverage for key security features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UDM SCAS test case to check if a synchronization failure message with an AUTS parameter containing an invalid MAC-S will be rejected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UDM SCAS tests may fail to detect a significant security flaw. This could result in attacks affecting the user authentication and privacy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TR 33.926 CR 0081 TS 33.102 CR 0283 TS 33.501 CR 1897 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[4]	3GPP TR 33.926 "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	3GPP TS 23.501: "System Architecture for the 5G System (5GS)".

[6]	3GPP TS 23.003: "Numbering, addressing and identification".

[7]	3GPP TS 33.102: "3G security; Security architecture".

[8]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

********** START OF 2nd CHANGE **********

#### 4.2.2.X	Authentication verification of synchronization failure messages

Requirement Name: Authentication verification of synchronization failure messages

Requirement Reference: TS 33.501 [2], clause 6.1.3.3.2 and TS 33.102 [7], clause 6.3.5.

Requirement Description:

-	“In case of a verification failure, the UDM/ARPF sends a Nudm_UEAuthentication_Get Response message with an authentication failure to the AUSF.” from TS 33.501 [2], clause 6.1.3.3.2.

-	“3.	If the verification failed, the HE/AuC sends an authentication failure response to the VLR/SGSN and it skips step 4-7.” from TS 33.102 [7], clause 6.3.5.

Threat References: TR 33.926 [4], clause E.2.2.X, Synchronization verification failure.

Test Case:

Test Name: TC_VERIFY_AUTH_SYNC_FAIL_MSG_UDM

Purpose:

Verify that the UDM validates the authentication of synchronization failure messages.

Procedure and execution steps:

Pre-Condition:

Test environment with an AUSF. The AUSF may be simulated.

Execution Steps:

1.	The tester generats the parameters RAND and AUTS in such a way that the MAC-S contained in AUTS is invalid, for example, by randomly choosing the MAC-S value.

2.	The tester sends an Nudm_UEAuthentication_Get message to the UDM with a "synchronisation failure indication" and the generated parameters.

3.	The UDM/ARPF performs steps 1-3 as described in TS 33.102 [7], clause 6.3.5.

Expected Results:

The UDM/ARPF rejects the AUTS parameter and sends a Nudm_UEAuthentication_Get Response message with an HTTP status code “403 Forbidden” and may include additional error information in the response body (in “ProblemDetails” element) as specified in TS 29.503 [8], clause 5.4.2.2.2, 2b.

NOTE:	The “cause” attribute of the “ProblemDetails” element may be AUTHENTICATION_REJECTED as described in TS 29.503 [8], clause 6.3.3.2.4.2-2.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of packet trace (e.g., pcap file).

### ********** END OF CHANGE **********