3GPP TSG-SA3 Meeting #115 	S3-240729

# Athens, Greece, 26th February - 1st March 2024						revision of S3-233855

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.523 | CR | 0002 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding the missing Xn-U interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_split_gNB |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Xn-U interface was missed when listing the interfaces to text. CR was not implemented when previously approved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the Xn-U interface plus correct capitialisation on ‘U’ and ‘C’ when refering to interfaces. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing interface mat not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.4, 5.4.4, 6.4.4, 7.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

### 4.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to gNB-CU.

The interface defined for the gNB-CU are in clause 4.2.3 of TS 23.501 [7] and in clause 4.1 of TS 38.300 [8].

According to clause 4.4.4 of TS 33.117 [2], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for gNB-CU, the following interfaces and protocols are in the scope of the testing:

For N2: the SCTP and NGAP protocols.

For N3: the UDP and GTP-U protocols.

For Xn-C: the SCTP and XnAP protocols.

For Xn-U: the UDP and GTP-U protocols.

For F1-C: the SCTP and F1AP protocols.

For F1-U: the UDP and GTP-U protocols.

NOTE: 	There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [2].

**** NEXT CHANGE ****

### 5.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to gNB-CU-CP.

The interface defined for the gNB-CU-CP are in clause 4.2.3 of TS 23.501 [7] and in clause 4.1 of TS 38.300 [8].

According to clause 4.4.4 of TS 33.117 [2], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for gNB-CU-CP, the following interfaces and protocols are in the scope of the testing:

For N2: the SCTP and NGAP protocols.

For Xn-C: the SCTP and XnAP protocols.

For F1-C: the SCTP and F1AP protocols.

For E1: the SCTP and E1AP protocols.

NOTE: There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [2].

**** NEXT CHANGE ****

### 6.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to gNB-CU-UP.

The interface defined for the gNB-CU-UP are in clause 4.2.3 of TS 23.501 [7] and in clause 4.1 of TS 38.300 [8].

According to clause 4.4.4 of TS 33.117 [2], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for gNB-CU-UP, the following interfaces and protocols are in the scope of the testing:

For N3: the UDP and GTP-U protocols.

For Xn-U: the UDP and GTP-U protocols.

For F1-U: the UDP and GTP-U protocols.

For E1: the SCTP and E1AP protocols.

NOTE: There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [2].

**** NEXT CHANGE ****

### 7.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to gNB-DU.

The interface defined for the gNB-DU are in clause 4.1 of TS 38.300 [7].

According to clause 4.4.4 of TS 33.117 [2], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for gNB-DU, the following interfaces and protocols are in the scope of the testing:

For F1-U: the UDP and GTP-U protocols.

For F1-C: the SCTP and F1AP protocols.

NOTE: 	There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [2].

**** END OF CHANGES ****