3GPP TSG-SA3 Meeting #115 	S3-240326

# Athens, Greece, 26 February - 1 March 2024

# Source:	KDDI Corporation

# Title:	New Key Issue on different cryptographic key lengths across handovers

# Document for:	Approval

Agenda Item:	5.5 New Study on enabling a cryptographic algorithm transition to 256-bits

# 1	Decision/action requested

Approve the pCR to TR 33.700-41

# 2	References

None

# 3	Rationale

This contribution proposes a new key issue for encouraging consistent use of 256-bit cryptography in the 5G System across handovers.

# 4	Detailed proposal

For SA3 to accept this proposal.

*** Start of 1st Change ***

## 5.X	Key Issue #X: Different cryptographic key lengths across handovers

### 5.X.1	Key issue details

As the 5G system transitions to 256-bit cryptographic algorithms, the situation may arise that a network deployment only partially supports 256-bits. That is, certain network elements are already upgraded to support 256-bit cryptographic algorithms, while others do not support them, yet, or will never support them.

In these scenarios, there is a risk of different key sizes being used for AS security to protect a single session as the UE moves through the network, depending on the gNB the UE is connected to. To illustrate the problem, assume that the UE is capable of 256-bit cryptographic algorithms:

- The UE is attached to a gNB that is capable of and is configured to prioritize 256-bit cryptographic algorithms. Therefore, a 256-bit cryptographic algorithm is selected.

- Next, the UE hands over to a gNB that either does not support or does not prioritize 256-bit cryptographic algorithms. Therefore, the algorithm agreed between the UE and the gNB is a 128 bit-algorithm.

- Next, the UE is again handed over, this time to a gNB that does support and is configured to prioritize 256-bit cryptographic algorithms. As such, a 256-bit cryptographic algorithm is selected.

As the above example illustrates, handovers can pose a challenge in such a mixed RAN deployment scenario: What is the expected handover behavior of UE and network? Is it possible to realize a uniform 256-bit cryptographic protection even if not all gNBs are upgraded and configured to support 256 bits or, alternatively, is it possible to have a mechanism that allows for more uniform algorithm strength across handovers to avoid unnecessary switching between algorithm strength either way?

### 5.X.2	Threats

Unless source gNB and target gNB both support the same algorithms, a change in algorithm strength may occur at every handover. This may be caused by a lack of support for 256-bit algorithms by the target gNB, meaning that it only supports legacy algorithms (Threat 3), or a misconfiguration of the target gNB, so that a 128-bit cryptographic algorithm is selected (Threat 4) in spite of support for 256-bit algorithms.

### 5.X.3	Potential security requirements

The 5G System shall have a mechanism to avoid unnecessary switching of key lengths across handovers based on operator policy.

*** End of 1st Change ***