3GPP TSG-SA3 Meeting #115	S3-240381

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0185 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to Robustness and Fuzz test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MITRE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clarifications to provide consistency, and objectivity of Robustness and fuzz testing test cases in TS 33.117 clauses 4.4.4. Reason for changes: Clause 4.4.4 - include testing for “malformed” input which is defined as “not conforming to a standard type” which is another test the fuzzing test tools are checking for. - For amount of time for fuzz or robustness testing “an amount of time sufficient to be effective” is subjective, in its simplistic form this can mean as little as one test. Propose to change this to “a coverage of tests sufficient to be effective” this is more in the spirit of fuzz testing because it requires a consideration for the range of tests performed, not only time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide clarifications on the testing tools, tool configuration, and procedures in Robustness and fuzz testing clauses 4.4.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Robustness and fuzz testing test cases are not clear and can produce different results for different test labs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* Start of 1st Change *************

### 4.4.4	Robustness and fuzz testing

Requirement Name: Robustness and fuzz testing

Requirement Reference: 4.2.6.2.2. – Interface Robustness

Requirement Description:

It shall be ensured that externally reachable services are reasonably robust when receiving unexpected or malformed input

Threat References: TR 33.926 [4]

Test case:

Test Name: TC_BVT_ROBUSTNESS AND FUZZ TESTING

Purpose:

To verify that the network product provides externally reachable services which are robust against unexpected or malformed input. The target of this test are the protocol stacks (e.g. diameter stack) rather than the applications (e.g. web app).

Procedure and execution steps:

Pre-Conditions:

-	The tester has the privileges to log in the network product and to access all system resources (e.g. log files)

-	A list of all available network services containing at least the following information shall be included in the documentation accompanying the Network Product:

-	all interfaces providing IP-based protocols;

-	the available transport layer protocols on these interfaces;

-	their open ports and associated services;

-	and a free-form description of their purposes.

NOTE: 	This list is to be validated as part of the BVT port scanning activity.

-	The robustness and fuzzing tools that are selected for this test shall utilize state-of-the-art technology to identify input which causes the Network Product to behave in an unspecified, undocumented, or unexpected manner.

-	Fuzz testing tools are a highly sophisticated technology and adaptation to the individual protocols in question is needed to be effective. Therefore, there is a lack of available effective fuzz testing tools available especially for protocols proprietary to the Telco industry. Taking into account note 4 of TR 33.916's clause 7.2.4, test labs shall acquire fuzz testing tools for those protocols where commercially feasible.

-	It needs to be taken into account that fuzz testing tools might show drastic differences in terms of effectiveness. The accredited test lab is expected to have sufficient expertise to recognize the level of effectiveness of the available tools.

-	A network traffic analyser on the network product (e.g. TCPDUMP) or an external traffic analyser directly connected to the network product and on a tester machine is available.

Execution Steps

The tester is required to execute the following steps:

1.	Execution of available effective fuzzing tools against the protocols available via interfaces providing IP-based protocols of the Network Product for a coverage of tests sufficient to be effective.

2.	Execution of available effective robustness test tools against the protocols available via interfaces providing IP-based protocols of the Network Product for aa coverage of tests sufficient to be effective.

3.	For both step 1 and 2:

a.	Using a network traffic analyser on the network product (e.g. TCPDUMP) or an external traffic analyser directly connected to the network product, the tester verifies that the packets are correctly processed by the network product.

b.	The testers verifies that the network product and any running network service does not crash.

c.	The execution of tests shall run sufficient times.

Expected Results:

A list of all of the protocols of the network product reachable externally on an IP-based interface, together with an indication whether effective available robustness and fuzz testing tools have been used against them, shall be part of the testing documentation. If no tool can be acquired for a protocol, a free form statement shall be used to explain why not.

The used tool(s) name, their unambiguous version (also for plug-ins if applicable), used settings, and the relevant output is evidence and shall be part of the testing documentation.

Any input causing unspecified, undocumented, or unexpected behaviour, and a description of this behaviour shall be highlighted in the testing documentation.

COTS fuzzing tools, by their nature, may have an acceptable failure rate (e.g. 0.1%) due to different non-deterministic variables in their implementation. At some point the tool’s documentation may even mention that the failing test shall be repeated to check whether it is really a recurring problem or not. The tester shall make best effort to determine if there is an issue with NE or the test tool and if necessary, work with the vendor of the network product to come to a consensus on the test result outcome.

Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

-	The used tool(s) name and version information,

-	Settings and configurations used

-    The output log file of the chosen tool that displays the results (passed/failed).

-	Screenshot

-	Test result (Passed or not)

-	Log/evidence tracing possible crashes

-	Any input causing unspecified, undocumented, or unexpected behaviour

************* End of 1st Change *************