3GPP TSG-SA3 Meeting #115 	S3-24xxxx

# Athens, Greece, 26 February -01 March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0057 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revocation procedure invoked by resource owner client |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPPY |  |  |  |  |  | Date: |  |  | 2024-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Address the EN for the revocation procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the revocation procedure triggered by the resource owner client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************** Start of changes ************************

#### 6.5.3.4	Revocation

API Exposing Function needs to be informed about revocation if this is necessary to ensure correct handling of revocation.

#### 6.5.3.4.X	Procedure for CAPIF revoking API invoker authorization initiated by resource owner client

Figure 6.5.3.4-1: Procedure for revoking API invoker authorization initiated by resource owner client

1.  The resource owner may trigger token revocation through the resource owner client. The resource owner client sends an authorization revocation request message to the CCF. The message shall include the service API ID, the GSPI, and other information related to the revoked token (e.g. the scope info).

2.	As in 8.23.4 of TS 23.222 [3]. In addtion, the request message also include the GSPI of the UE.

3.  As in 8.23.4 of TS 23.222 [3]. In addtion, the AEF may determine whether to update the resource due to the revocation, e.g. for the QoS service API, the AEF may inform PCF/SMF to modify the QoS level of correspoinding PDU sessions after revocation.

4.  As in 8.23.4 of TS 23.222 [3].

5.  As in 8.23.4 of TS 23.222 [3]. The difference is that the invalidated authorization is for the resource owner/UE corresponding to the GSPI.

6.  As in 8.23.4 of TS 23.222 [3]. The difference is the message is sent to the resource owner client.

******************************* End of changes *********************************