3GPP TSG-SA3 Meeting #115 	S3-240500

Athens, Greece, 26 February -01 March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1920 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the EN in the X.9 Authorization of selection of participant NWDAF instances in the Federated Learning group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA _Ph3_SEC |  |  |  |  |  | Date: |  |  | 2024-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This proposal is to remove the EN in the X.9 authorization of selection of participant NWDAF instances in the Federated Learning group. “Editor’s note: The use of Service area and Availability time requirement for authorization is FFS.” As defind in the TS23.288 clause 5.3, “The NWDAF containing MTLF further determines whether the ML model should be trained via FL mechanism based on Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to data privacy, data security).” In this case, the FL client has some data which is not allowed sharing with NFp. In other words, if the NWDAF can collect data out of its service area, that means the NWDAF can collect all data in the core network and train model alone, and there is no need to support Federated learning any more. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to remove the EN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The data security and data access right can not be protected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

X.9	Authorization of selection of participant NWDAF instances in the Federated Learning group

The authorization for selecting participant NWDAF instances in the Federated Learning (FL) group uses token-based authorization as specified in clause 13.4.1, with the following additions.

Figure X.9-1 depicts the authorization mechanism for NWDAF containing MTLF acting as FL Server to initiate the Federated Learning process on the NWDAF containing MTLF(s) acting as FL Client(s). The authorization is based upon the FL capability type (FL server or FL client) and Service area provided by the NWDAF containing MTLF acting as FL server during registration, and the Analytics ID and Interoperability Indicator per Analytics ID provided by the NWDAF containing MTLF acting as FL client during registration.

Figure X.9-1: FL Authorization for selecting participant NWDAF instances

Step 1a. The NWDAF containing MTLF acting as FL client registers to the NRF with its FL related information, including supported FL capability (FL client), Analytics ID(s) and Interoperability Indicator per Analytics ID as described in clause 5.2 of TS 23.288.

Step 1b. The NWDAF containing MTLF acting as FL server registers to the NRF with its FL capability (FL Server).

Step 2. The NWDAF containing MTLF acting as FL server (NF Service Consumer) sends a discovery request to NRF and receives the available NWDAFs containing MTLF acting as FL client(s) (NF Service Producer) as a response, as specified in clause 6.2C.2.1 of TS 23.288 [105].

Step 3. The NWDAF containing MTLF acting as FL server (NF Service Consumer) sends an access token request to the NRF as specified in clause 13.4.1. The access token request may contain the Analytics ID and Area of Interest for the requested Federated Learning process.

Step 4. The NRF authorizes the NWDAF containing MTLF acting as FL server (NF Consumer) based upon the information received in Step 1b, and after verifying that the Server NWDAF’s Vendor ID is included in the Interoperability Indicator for the requested Analytics ID provided in Step 1a. If the authorization succeeds, NRF generates the access token(s) as specified in clause 13.4.1. The access token claims may include the Analytics ID and Area of Interest for the request Federated Learning process.

NOTE: Fine-grained authorization can be done locally at the NWDAFs containing MTLF acting as FL client(s) (NF Service Producer).

Step 5a, 5b. The NRF sends the access token to the NWDAF containing MTLF acting as FL Server, or rejects the request in case of failed authorization, as described in clause 13.4.1.

Step 6. The NWDAF containing MTLF acting as FL server sends the service request to the NWDAF(s) containing MTLF acting as FL client with the access token received in Step 5a. along with the Analytics ID information for which the FL process is to be performed, as described in TS 23.288 [105].

Step 7, 8. The NWDAF containing MTLF acting as FL client (NF Service Producer) verifies the received access token as specified in clause 13.4.1. In case of successful access token verification, the NWDAF containing MTLF acting as FL client sends a success response to the NWDAF containing MTLF acting as FL server, as described in TS 23.288 [105].

Step 9. After a successful response from the NWDAF(s) containing MTLF acting as FL client, the NWDAF containing MTLF acting as FL server initiates the Federated Learning process as described in TS 23.288 [105].

Authorization of the NWDAF containing MTLF acting as FL client is implicit, since it can join a Federated Learning group only when selected by the NWDAF containing MTLF acting as FL server.