3GPP TSG-SA3 Meeting #115	S3-240675

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.210 | CR | 0079 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the IKEv2 profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CryptoSP |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - AES-CBC was removed as mandatory to implement in Rel-15. AES-CBC was intended to be used with HMAC-SHA256. Mandatory support of AUTH_HMAC_SHA256_128 is not necessary anymore. Note that confidentiality is mandatory in IKEv2 but optional in ESP. - BSI TR-02102-3 states that 2048-bit MODP group is only for use up until 2022. NIST and ANSSI only allow 2048-bit Finite Field Diffie-Hellman if the data does not have to be protected after 2030.The is no reason to support or use 2048-bit MODP as the strong Diffie-Hellman group 19 has been mandatory to support since Rel-13. - As stated in RFC 9206 and NIST SP 800-56A there are security concerns with reuse of Diffie-Hellman private keys. - RFC 9206 clarifies that Identification Payloads must not be used for authentication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Mandatory support of AUTH_HMAC_SHA256_128 is removed. - Support of 2048-bit MODP Groups is forbidden. - Reuse of Diffie-Hellman private keys is forbidden. - Clarification that Identification Payloads must not be used for authentication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistent profile as AUTH_HMAC_SHA256_128 without a legacy non-AEAD encryption algorithm. - The weak 2048-bit MODP Groups might be used in violation with BSI recommendations. - Implementations might reuse Diffie-Hellman private keys which has security concerns. - Implementations might use Identification Payloads for “authentication”, i.e., there is not authentication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ******* FIRST CHANGE *************

### 5.4.2	Profiling of IKEv2

The Internet Key Exchange protocol IKEv2 shall be supported for negotiation of IPsec SAs. The following additional requirements apply.

General:

IKEv2 Configuration Payload as defined in RFC 7296 [43] should be supported.

Protocol support for High Availability as defined in RFC 6311 [42] should be supported.

An ephemeral private key shall be used in exactly one key establishment transaction and shall be destroyed (zeroized) as soon as possible.

For IKE_SA_INIT exchange:

The following algorithms are listed with their names according to [44].

Following algorithms shall be supported:

-	Confidentiality: AES-GCM with a 16 octet ICV with 128-bit key length;

-	Pseudo-random function: PRF_HMAC_SHA2_256;

-	Diffie-Hellman group 19 (256-bit random ECP group) ;

Following algorithms should be supported:

-	Confidentiality: AES-GCM with a 16 octet ICV with 256-bit key length;

-	Pseudo-random function: PRF_HMAC_SHA2_384;

-	Diffie-Hellman group 20 (384-bit random ECP group).

-	Diffie-Hellman group 31 (Curve25519).

NOTE 1:	 The IANA IKEv2 registry [44] contains further references for the algorithms listed.

For security reasons, the use of Diffie-Hellman MODP groups less than 3072-bit shall not be supported.

For IKE_AUTH exchange:

-	Authentication method 2 - Shared Key Message Integrity Code shall be supported;

-	IP addresses and Fully Qualified Domain Names (FQDN) shall be supported for identification;

-	Re-keying of IPsec SAs and IKE SAs shall be supported as specified in RFC 7296 [43].

-	In addition to the requirements defined in RFC 7296 [43], rekeying shall not lead to a noticeable degradation of service.

-	Identification Payloads (IDi and IDr) shall not be used for the IKEv2 authentication but may be used for policy lookup.

For the CREATE_CHILD_SA exchange:

-	A DH key exchange should be used (giving Perfect Forward Secrecy) and the session keys should be changed frequently.

For reauthentication:

-	Reauthentication of IKE SAs as specified in RFC 7296 [43] section 2.8.3 shall be supported;

-	A NE shall proactively initiate reauthentication of IKE SAs, and creation of its Child SAs, i.e. the new SAs shall be established before the old ones expire;

-	A NE shall destroy an IKE SA and its Child SAs when the authentication lifetime of the IKE SA expires;

NOTE 2:	NE actions related to reauthentication are controlled by locally configured lifetimes according to RFC 4301 [35]: a soft authentication lifetime that warns the implementation to initiate reauthentication, and a hard authentication lifetime when the current IKE SA and its Child SAs are destroyed.

-	In addition to the requirements defined in RFC 7296 [43], reauthentication shall not lead to a noticeable degradation of service.

## ******* END OF CHANGES ************