3GPP TSG-SA3 Meeting #115	S3-240694

# Athens, Greece, 26th February - 1st March 2024			     (revision of xx-yyxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Security for Selective SCG Activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This draftCR aims to clarify the SCPAC procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the detailed security mechanism and procedures for subsequent CPAC (SCPAC). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The security protection for the subsequent CPAC (SCPAC) procedure is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.10.2.X	Security mechanism and procedures for SCPAC

### 6.10.2.X.1		General

In subsequent CPAC (SCPAC), the MN may provide one or several candidate SCG configuration(s) for one or multiple candidate SN(s) to the UE. The UE may select and execute precisely one of these conditional reconfigurations to change PSCell based on the measurement results on candidate target PSCells. The conditional reconfiguration for the selected PScell remains valid after the UE selects the target and executes the target cell access procedure. Thus, the UE can connect to the same SN several times without any further reconfiguration by the network.

Figure 6.10.2.X.1-1: Security mechanism and procedures for SCPAC

### 6.10.2.X.2		Security context initialization for selective SCPAC

To prevent key-stream reuse when the UE switches back and forth to the same PSCell or the SN, the MN shall assign a sequence of multiple distinct SN Counter values (maintained for dual connectivity) per candidate SN during the subsequent CPAC (SCPAC) procedure. Each SN Counter value is unique, and the sequences are non-overlapping. These SN Counter value sequences per SN shall be provided to the UE by the MN. The UE shall store these sequences.

The MN shall derive corresponding KSN keys as described in Annex A.16 based on the corresponding SN Counters values for each target SN. KSN keys and the corresponding SN counter values are sent to the SN from the MN in the SN Addition Request. The SN shall store the KSN keys and the corresponding SN Counter value sequence in its security context. The MN shall maintain the largest assigned SN Counter value and monotonically increment it for the next KSN calculation for DC as described in subclause 6.10.3.1 of this document or for further assignment for the SCPAC detailed in this clause.

When a new AS root key, KNG-RAN, in the associated 5G AS security context is established, and the SN Counter is set to ‘0’ as specified in clause 6.10.3.1, the MN assigns a new sequence of distinct SN Counter values per candidate SN and sends these to the UE in the RRC Reconfiguration. The UE shall delete the stored KSNs and SN Counter value sequence and store the received new SN Counter values. Further, the MN derives the corresponding KSN for each target SN, and the derived KSN keys and the corresponding SN Counter value sequence are sent to the SN from the MN. Each SN shall delete the stored KSNs and corresponding SN Counter values and store the received new KSNs and the corresponding SN Counter values.

### 6.10.2.X.3		Security mechanism for UE to access target PSCell or SN

When the UE accesses an SCG of a target PSCell or SN for the first time or a subsequent time based on the same conditional reconfiguration, the UE shall select the first unused SN Counter value in order among the stored ones associated with the target PSCell or SN. Because all counter values are distinct, selecting the first unused one ensures that it is not previously used with the current KgNB. The UE shall then derive the corresponding KSN using the SN Counter as described in Annex A.16 of this document and shall initiate the access procedure. The SN chooses the first unused KSN key of the UE to establish the security association with the UE.

In parallel, UE shall informs the MN of the SN Counter value utilized for KSN derivation in the RRC Connection Reconfiguration Complete. The MN, in turn, shall relay the corresponding SN Counter value to the SN in the SN Reconfiguration Complete message.

The SN shall use the received SN Counter value to avoid the key mismatch. Since the integrity protection is optional to use, from the received user plane packet it is not possible for the SN to determine the key mismatch if integrity protection is not applied. The SN upon receiving the SN counter value from the UE via the MN, shall check whether the corresponding SN Counter value of the chosen KSN is same as the received SN Counter value to determine the KSN mismatch. In case of a KSN mismatch, the SN, having stored the KSN keys and the corresponding SN Counter values, selects the appropriate KSN based on the received SN counter values for subsequent data access under the same reconfiguration.

# * * * * Start of Second change * * * *

# A.16	Derivation of KSN

This input string is used when the MN and UE derive KSN. The following input parameters shall be used:

-	FC =0x79,

-	P0 = Value of the SN Counter as a non-negative integer,

-	L0 = length of the SN Counter value (i.e. 0x00 0x02).

The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB.

# * * * * End of changes * * * *