3GPP TSG-SA3 Meeting #115 	S3-240985

# Athens, Greece, 26th February – 1st March 2024	Revision to draft_S3-240367-r2

# Source:	Johns Hopkins University APL, Cisco Systems, AT&T

# Title:	New key issue on Certificate Enrolment

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

It is requested to approve a key issue for TR 33.776

# 2	References

[1]	SP-231787, New Study of ACME for Automated Certificate Management in SBA, 3GPP SA#102

[2]	draft_S3-240207, Proposed skeleton for TR 33.776 Study of Automated Certificate Management Environment (ACME) for the Service Based Architecture (SBA)

[3]	IETF RFC 8555, Automatic Certificate Management Environment (ACME), March 12, 2019

# 3	Rationale

As the use of digital certificates and virtualized environments expand in the 5GC, resources needed to manually manage certificates can become quite intensive and could lead to increase likelihood of misconfiguration, vulnerabilities, and unexpected downtime. There are benefits for enrolment of digital certificates for such entities as operator CAs and 5GC NFs that are automated, secure, scalable and interoperable with certificate management protocols. Therefore, certificate enrolment that is automated and fully interoperable with ACME in the 5GC SBA should be supported [1].

Certificate enrolment is the process of obtaining a digital certificate from a CA. The process involves authentication, request submission, certificate generation, reception and installation. For this key issue for FS_ACME_SBA [2], the procedures and solutions for a NF and ACME client [3] to interoperate with the enrolling CA are required to support ACME in the 5GC SBA.

# 5	Key issues

****Start of Change All New Text****

## 5.X		Key Issue: Certificate enrolment

## 5.X.1 	Key Issue Details

The ACME automated certificate management protocol provides procedures and identifies solutions to support authentication to the enrolment server CA and secure message protocol to protect ACME message exchanges during the certificate enrolment process against replay and confidentially protection. To address the objectives of this study [1] there is a requirement to identify procedures and solutions to use ACME across the 5GC SBA for different scenarios (e.g., multi-vendor integration) and use cases (e.g., authentication of domain names, HTTPS, mutual TLS authentication). Procedures and solutions for automated certificate enrolment to consider for this key issue include:

- Support for ACME client and authentication

- Certificate signing request (CSR) – content and creation of request

- CSR Submission

- Certificate issuance

This KI is to identify ACME certificate enrolment procedures and solutions for different use cases for the 5GC SBA.

### 5.X.2 	Security Threats

Not Applicable.

### 5.X.3	Potential Security Requirements

Not Applicable.

****End of Change****