3GPP TSG-SA3 Meeting #116	S3-241730

Jeju, South Korea,  20th - 24th May 2024

# Source:	3GPP CT4

# Title:	Reply LS on Authorization of NF service consumer for data collection via DCCF

# Document for:	Information, Discussion

Agenda Item:                3

3GPP TSG-CT WG4 Meeting #121	C4-240623

# Athens, Greece; 26th Feb – 1st March 2024

# Title:	Reply LS on Authorization of NF service consumer for data collection via DCCF

# Response to:	LS (C3-235594) on Authorization of NF service consumer for data collection via DCCF from CT3

# Release:	Rel-17

# Work Item:	eNA_Ph2

Source:	CT4

To:	CT3

Cc:	SA3, SA2

Contact Person:

Name:	Bruno Landais

Tel. Number:

E-mail Address:	Bruno.landais@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C4-240136 [29.500 CR on Authorization of NF Service Consumers for data access via DCCF]

1. Overall Description:

CT4 thanks CT3 for their LS (C3-235594) on Authorization of NF service consumer for data collection via DCCF.

Clause X.2 of TS 33.501 specifies how to handle the case where a new Data Consumer requests the DCCF to access data for which the DCCF already created a subscription to the NF Service Producer:

NOTE 6: In the case a new NF Service Consumer comes at a later stage to request the data, which is already being collected by DCCF, steps 1-10 apply. When the request is received by the NF Service Producer (i.e. the data producer), it authenticates the NF Service Consumer and verifies the access token provided along with the service request and sends to DCCF the access token verification response. DCCF based upon the response received, either updates the subscription info to include the new NF Service Consumer as well and sends the data to both the consumers (as specified in Clause 6.2.6.3.2 in TS 23.288 [105]), or in the case of access token verification failure, the DCCF rejects the request received by the NF Service Consumer.

Accordingly, CT4 has agreed the attached 29.500 CR clarifying how a Data Source (NF Service Producer) authorizes a new Data Consumer (Source NF Instance), when a new Data Consumer requests the DCCF to access data for which the DCCF already created a subscription to the NF Service Producer.

2. Actions:

To CT3 group.

ACTION: 	CT4 asks CT3 group to take the above information into account.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm