3GPP TSG-SA3 Meeting #116 	S3-241771

Jeju, Korea 20-24 May 2024

# Source:	OTD_US

# Title:	New SID on security aspects of roaming traffic offload via home-session breakout

# Document for:	Approval

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on security aspects of roaming traffic offload via home-session breakout

Acronym:	FS_HSBO_Sec

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | x |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

| x | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1030018 | Roaming traffic offloading via session breakout in HPLMN | SA2 TEI19 WID: TEI19_HSBO |

# 3	Justification

By offloading traditionally home-routed traffic locally within a visited country utilizing a locally deployed H-UPF PDU session anchor hosted in a 3rd party data centre, there are potential additional security requirements to be placed on the V-UPF to H-UPF (modified N9), V-SMF to H-SMF (modified N16), and H-SMF to H-UPF (cross-domain N4) which need to be studied before SA2 agrees on an architecture and session establishment procedure.

NOTE: Current architecture in TS 23.501 assumes for 5G roaming with local breakout that the N6 is within the VPLMN and N32 proxies between the VPLMN and HPLMN.

Example Architecture from SP-240491 (S2-2403552):

Figure 1. Traffic offloading at the local H-UPF controlled by H-SMF in HR roaming scenario

Whether the V-UPF will accept foreign network direct connection over N9 without a proxy controlling the resource request is an issue. Secondary issue relates to whether there is a local PSA (L-PSA) within the VPLMN.

SA2 has identified the following high level Work Tasks:

- WT 1:	Specify how to select a proper H-UPF deployed in the visited region of the serving PLMN for HR PDU session supporting roaming traffic offloading at HPLMN.

- WT 2:	Specify how to trigger change of H-UPF in order to use a H-UPF in the home country when user plane path failure is detected between the H-UPF deployed in the visited region of the serving PLMN and the V-UPF.

# 4	Objective

This study has the following objectives/Work Tasks:

WT1: Study whether functional or security related modifications are needed to the SEPP when N32 is routed through an intermediary cloud provider and not across the IPX.

WT2: Study privacy and security impacts of N4 extension from the HPLMN to 3rd Party Data Center domain.

WT3: Study whether there are additional topology exposure risks associated with 3rd party intermediary services hosting a H-UPF instance.

WT4: Study whether additional PRINS functionality is needed at the roaming hub/cloud data center if the cloud data center is connected to multiple foreign networks within the VPLMN.

WT5: Study potential impacts to the existing IPUPS (inter PLMN N9 Security) solutions.

WT6: Study potential privacy and security impacts of modified N6 between an H-UPF (PSA) and local DN in a visited country or N6 between H-UPF in visited country and home country DN.

NOTE: Additional privacy and security aspects related to SA2 Work Tasks can be addressed based on SA2 progress.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT1 | .5 | .5 | No | No |
| WT2 | .5 | .5 | No | No |
| WT3 | .5 | .5 | No | No |
| WT4 | .5 | .5 | No | No |
| WT5 | .5 | .5 | No | No |
| WT6 | .5 | .5 | No | No |

Total TU estimates for the study phase:     3

Total TU estimates for the normative phase:    3

Total TU estimates: 6

# 5	Expected Output and Time scale

| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| --- | --- | --- | --- | --- | --- |
| Internal TR | 33.7xy | Study on security aspects of roaming traffic offload via home-session breakout | SA#106 (December 2024) | SA#107 (March 2025) |  |

# 6	Work item Rapporteur(s)

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

System architectural aspects will be covered by SA2.

Lawful Interception aspects will be covered by SA3-LI.

# 9	Supporting Individual Members

Supporting IM name

OTD_US

US National Security Agency

Johns Hopkins University APL

MITRE Corporation