3GPP TSG-SA3 Meeting #116	S3-241785

# Jeju, Korea (Republic Of), 20th May 2024 - 24th May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1980 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Eliminate the possibility of implementing unverified security protocols for the UDR-UDM interface. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current note suggests that UDM-UDR interfaces allow the use of implementation-specific protocols for transport security. This interface transports highly sensitive information, such as the long term persistent keys of each subscriber. The use of implementation-specific algorithms carries the risk of using a theoretically unverified security algorithm, which must be prevented at all costs. Therefore, we suggest deleting this part of the note. It is important to note that in any case the rules of section 13.1 of 33.501 apply, which allows the implementation of security protocols such as TLS or IPsec. In this context, we also refer to the note 1-3, which allows specific rules of the use of TLS and IPsec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Edit a note that may introduce unverified security protocols for the UDM-UDR interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Vendors may implement insecure protocols that transport highly sensitive information, such as each participant's long-term persistent keys. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3-240723 |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

### 5.8.1	Generic requirements

The long-term key(s) used for authentication and security association setup purposes shall be protected from physical attacks and shall never leave the secure environment of the UDM/ARPF unprotected.

NOTE 1: Security mechanisms for protection of subscription credentials in ARPF are left to implementation.

NOTE 2: Security mechanisms for storage of subscription credentials in the UDR (as specified in 3GPP TS 29.505 [70]) are left to implementation.

********** END OF CHANGE **********