3GPP TSG-SA3 Meeting #116 	S3-241894

# Jeju, Korea, 20-24 May 2024

# Source:	Apple

# Title:	New solution on correctly indicating the security capability

# Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

Approve the pCR to TR 33.700-41

# 2	References

None

# 3	Rationale

This contribution proposes a new solution for TR 33.700-41.

# 4	Detailed proposal

*** Start of 1st Change ***

## 6.Y	Solution #Y: Correctly indicating the security capability

### 6.Y.1	Introduction

This solution addresses the security requirement in key issue#X on correctly indicating the security capability.

### 6.Y.2	Solution details

During the registration request, when UE reports its security capability, if UE supports the 3 new 256-bit algorithms, UE uses 5G-EA4/5/6, 5G-IA4/5/6 in UE security capability IE (refer TS 24.501 Figure 9.11.3.54.1) to indicate to the network.

Those indication can also be used by AMF and gNB to indicate the new algorithms in NAS SMC procedure and AS SMC procedure and handover procedure.

During handover procedure, AMF use the 5G-EA4/5/6, 5G-IA4/5/6 to indicate the UE’s security capability to the target AMF.

5G-EA4: AES based 256 bits confidentiality algorithm

5G-EA5: SNOW 5G based 256 bits confidentiality algorithm

5G-EA6: ZUC based 256 bits confidentiality algorithm

5G-IA4: AES-256 based 256 bits integrity algorithm

5G-IA5: SNOW 5G based 256 bits integrity algorithm

5G-IA6: ZUC based 256 bits integrity algorithm

Fig 1: (refer TS 24.501 Figure 9.11.3.54.1) UE security capability information element

### 6.Y.3	Evaluation

This solution addresses the security requirement in key issue#x with little impact, UE, gNB and AMF should be updated the coding point accordingly based on its capability.

*** End of 1st Change ***