3GPP TSG-SA3 Meeting #116	S3-241945

Jeju, South Korea, 20th - 24th May 2024

Title:	Reply-LS on PQC Migration

Response to:	LS S3-240692 on 3GPP studies for PQC Migration from GSMA

Release:

Work Item:

Source:	SA WG3

To:	GSMA PQTN

Cc:

Contact person:	lei.zhongding@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA3 thanks GSMA for the LS on 3GPP studies for migration to Post-Quantum Cryptography (PQC). SA3 would like to provide the following responses to the raised questions:

- Q1: Timeline of the study, specifications and migration for both symmetric algorithms and asymmetric algorithms, cryptographic primitives, and relevant protocols

[SA3]: SA3 completed one study on impact of quantum computing to 3GPP systems back in 2018. The outcomes were captured in the report TR 33.841: Study on the support of 256-bit algorithms for 5G (Release 16). There has been no further studies or specifications developed focusing on PQC since then. In general, SA3 has the following understanding on the PQC issues:

- quantum computing is likely to undermine the security of all asymmetric algorithms in common use today, including those used in the profiles and protocols in the 5G systems, e.g. ECIES for SUPI encryption, IPsec, TLS, DTLS, JWS etc. These algorithms, protocols, or profiles will be updated to quantum-safe versions once these are available. It is notable that all the asymmetric algorithms are developed in other standard bodies. They are assessed before being adopted or adapted into 3GPP systems.

- As far as the symmetric algorithms are of concern, all the 128-bit algorithms currently used in 5G systems are considered secure against quantum computing. Besides, a number of 256-bit algorithms have been specified to offer higher security options, e.g. TS 35.240-248 for Snow 5G/AES/ZUC algorithms (Release 18).

- Q2: Are the legacy systems i.e., 4G, 3G etc., considered for PQC migration? If so, then what is the timeline for the migration of legacy systems

[SA3]: SA3 has not made any decisions on whether a legacy system, e.g. 4G or 3G, is required for PQC migration. However, SA3 has discussed the issue on the following aspects:

- Symmetric algorithms (128 bit): they are used to secure the air interface as well as traffic within/between networks, e.g. traffic protection in IPsec. These symmetric algorithms are considered secure against quantum computing in foreseeable life time of the legacy systems.

- Asymmetric algorithms/protocols/profiles (e.g. IPsec, TLS): they are used for 3GPP network domain security in the legacy systems. SA3 will continue to discuss the needs for PQC migration.

In the meantime, SA3 is actively monitoring the status of quantum computing and development of PQC algorithms / protocols in other SDOs.

# 2	Actions

To GSMA

ACTION: 	SA3 kindly asks GSMA to take above information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#117	19 - 23 August 2024		Maastricht (Netherlands)

SA3#118	14 - 18 October 2024		TBD (India)