3GPP TSG-SA3 Meeting #116	S3-241959

Jeju, South Korea, 20th - 24th May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.256 | CR | 0049 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to address DoS attacks to UAV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the architecure model in TS 23.256, a UAV is a “2-in-1” UE, i.e. a normal UE together with an arieal service subscription. The UAV (UE) can receive UAV services only after a successful UAV USS authentication and authorization (UUAA). If UUAA is faild, the UAV (UE) can only be provided with services for “normal UE” (i.e. non-UAV services). This model may pose potential security risks to the UE. For example, a malicious third-party AF may request the UAV (UE) to establish excessive PDU sessions or feed the UAV (UE) with large amount of data traffic. These events would pose serious security or safety hazards especially when the UAV is in flight since the UAV’s battery may be depleted and its UAV services may be interrupted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a security requirement to UAV. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UAV may be vulnerable and subject to malicious attacks |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************** Start of changes ************************

### 5.2.1	UUAA in 5GS

#### 5.2.1.1	General

The UAV USS authentication and authorization (UUAA) is the procedure to ensure that the UAV can be authenticated and authorized by a USS before the connectivity for UAS services is enabled. This clause specifies the relationship between primary authentication (as described in clause 6.1 in TS 33.501 [2]) and UUAA. An UAV is allowed to perform UUAA with the USS/UTM only after the UAV (UE) has completed successfully primary authentication.

It may be triggered by the AMF when UAV is registering with 5GS or triggered by the SMF during the PDU session establishment procedure. The UUAA procedure may also be triggered by a USS for re-authentication if the USS had authenticated the UAV. Network support for UUAA during registration is optional while it is mandatory during the PDU Session establishment. UE Support for UUAA during registration and during the PDU Session establishment is mandatory.

The AMF or SMF triggers the UUAA procedure if the UAV has an Aerial UE subscription and the UAV requests access to UAS services by providing the CAA-Level UAV ID of the UAV in the Registration Request or PDU Session Establishment Request.

The UUAA is performed between the UAV and the USS. The UAV is authenticated based on the CAA-Level UAV ID and credentials associated to the CAA-Level UAV ID. The authentication messages are included in a transparent container and conveyed between the UAV and the USS via a 3GPP UAS NF.

NOTE:	The provision of CAA-Level UAV ID, credentials, and the actual authentication methods and information that needs to be sent to perform the UUAA are out of scope of the 3GPP specifications.

On successful completion of a UUAA, the USS can send UAS security information in the UUAA Authorization Payload to the UAV. The contents of that security information are out of scope of the 3GPP specifications.

After successful completion of a UUAA, a PDU session establishment procedure corresponding to a DN not for aerial services shall not be triggered by the network through an AF or by the UAV.

The UUAA procedure at registration in 5G is described in the clause 5.2.1.2 and the UUAA procedure during PDU session establishment procedure is described in the clause 5.2.1.3.

At any time after the initial registration, the USS or the AMF (when the networking supports UUAA during registration) may initiate the Re-authentication procedure for the UAV. The AMF initiated Re-authentication procedure is described in the clause 5.2.1.2, whereas the USS initiated Re-authentication procedure is described in the clause 5.1.2.4.

Figure 5.2.1.1-1 provides an example of how UUAA fits into the 5GS procedures. The complete description of this flow is given in TS 23.256 [3].

Figure 5.2.1.1-1: UUAA in 5GS

1.	The UE sends a Registration Request message to the AMF. The UE may provide a CAA-Level UAV ID, and optionally a USS address/IP address, to indicate the request is registering for UAS services. In case the CAA-Level UAV ID and/or USS address/IP address is configured not to be sent in plain text, e.g., the USS address or an IP address not to be exposed in public, the CAA-Level UAV ID, and USS/IP address if available, shall be sent after the NAS security is established.

2.	AMF completes security set up including primary authentication as needed.

3.	After successful Primary authentication, AMF determines whether UUAA is required for the UE. UUAA shall only be triggered if the UE has provided a CAA-Level UAV ID and has a valid Aerial UE subscription. AMF may skip UUAA if the UE has completed UUAA successfully before and the UE UUAA is current, i.e., the UE's authentication and authorization has not been revoked after a previous successful UUAA.

4a. AMF shall return a Registration Accept message to the UE and indicate that UUAA is pending.

4b.	UE may send a Registration Complete message to acknowledge the AMF.

5. AMF triggers the UUAA procedure if determined needed in step 3 as described in clause 5.2.1.2.

The following procedure is for UUAA during PDU session establishment:

6. The UE sends a PDU Session Establishment Request message to the SMF including a CAA-Level UAV ID to indicate the request is for UAS services.

7. The SMF determines whether UUAA is required for the UE. UUAA shall only be triggered if the UE has provided a CAA-Level UAV ID and has a valid Aerial UE subscription. SMF may skip UUAA, the UE has completed UUAA successfully with the same USS/DN before, i.e., at registration as in step 5 or in previous PDU Session Establishment procedures and a successful UUAA result is available.

8. The SMF triggers the UUAA procedure if determined needed at step 7 as described in clause 5.2.1.3.

******************************* End of changes *********************************