3GPP TSG-SA3 Meeting #116	S3-242024

Jeju, South Korea,  20th - 24th May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | 0095 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections and additions for the N3IWF network product class |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CAICT, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The annex M for the N3IWF network product class annex is not inline with the common skeleton for such clauses. The clause is expected to properly describe the security problem for the target class including for example the critical assets. This is currently missing and hence incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of missing clauses to the annex M for N3IWF network product class. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete security problem description for the N3IWF network product class. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x]	3GPP TS 33.520: " Security Assurance Specification (SCAS) for Non-3GPP InterWorking Function (N3IWF)".

**** Next Changes****

Annex M:
Aspects specific to the network product class N3IWF

M.1	Network product class description for the N3IWF

M.1.1	Introduction

This Annex covers the aspects specific to the N3WF network product class.

M.1.2	Minimum set of functions defining the N3IWF network product class

As part of the N3IWF network product, it is expected that the N3IWF to contain N3IWF application, a set of running processes (typically more than one) executing the software package for the N3IWF functions and OAM functions that is specific to the N3IWF network product model. Functionalities specific to the N3IWF network product introduce additional threats and/or critical assets as described below. Related security requirements and test cases have been captured in TS 33.520 [x].

NOTE:	For the purposes of the present document, this common set is defined to be the list of N3IWF functions contained in clause 6.2.9 of 3GPP TS 23.501 [8].

M.2	Assets and threats specific to the N3IWF

M.2.1	Critical assets

In addition to the critical assets of a GNP described in clause 5.2 of the present document, the critical assets specific to the N3IWF to be protected are:

-	N3IWF Application;

-	Mobility and Session Management data: e.g. subscriber's identities (e.g. SUCI), subscriber keys (I.e. KN3IWF), APN name, data related to mobility management like UE status, UE's IP address, etc., session management like PDN type, QoS and so on, etc.

-	The interfaces of N3IWF to be protected and which are within SECAM scope:

-	Reference point interfaces:

-	N2.

-	N3.

-	Y2.

-	Console interface, for local access: local interface on N3IWF

-	OAM interface, for remote access and data collection: interface between N3IWF and OAM system

NOTE 1: 	The detailed interfaces of the N3IWF class are described in clause 4, Network Product Class Description of the present document.

-	N3IWF Software: binary code or executable code

NOTE 2: 	N3IWF files may be any file owned by a user (root user as well as non-root uses), including User account data and credentials, Log data, configuration data, OS files, cryptographic material, N3IWF application, or N3IWF Software.

M.2.2	Threats related to EAP procedure

-	Threat name: N3IWF sends EAP-Identity Request

-	Threat Category: Denial of service.

-	Threat Description: EAP-5G is used between UE and N3IWF. As specificed in TS 33.501[14], the N3IWF shall refrain from sending an EAP-Identity request. The UE may ignore an EAP Identity request or respond with the SUCI it sent in the Registration Request. This means if the N3IWF happens to send an EAP-Identity Request to the UE, the N3IWF shall not look forward an EAP-Identity Reply. This is different from normal EAP framework. If the N3IWF behaves the same as normal EAP framework, the N3IWF will wait for a reply till time expires. This may casue the UE cannot access to the network via an N3IWF.

-	Threatened Asset: GNP services.

**** End of Changes****