3GPP TSG-SA3 Meeting #116	S3-24xxxx

Jeju, South Korea, 20th - 24th May 2024

————————————————————————————————————————————————

Source:	Huawei, HiSilicon

Title:	New key issue on security aspect of digital asset container in 5GC

Document for:	Approval

Agenda Item:	5.18

- 1	Decision/action requested

Approve the new KI for inclusion in TR 33.721 [1]

- 2	References

[1]		3GPP TR 33.721 ' Study on security aspects of 5G Mobile Metaverse services'

- 3	Rationale

This contribution proposes a new key issue about the digital asset container security. The issues was proposed in the WT #2 in S3-240974.

Several requirements on digital asset management have been agreed in SA1 and consolidated in TS 22.156:

[R-5.2.3-001] Subject to operator policy, regulatory requirements and user consent, the 5G system shall be able to provide functionality to store digital assets associated with a user, and to remove such digital assets associated with a user.

[R-5.2.3-003] Subject to operator policy and user consent, the 5G system shall be able to allow an authorized third party to retrieve the digital asset(s) associated with a user, e.g., when the user accesses a specific application.

[R-5.2.3-006] Subject to operator policy, regulatory requirements and user consent, the 5G system shall support a mechanism for users to define conditions (e.g., based on user location information) to restrict the access to, and management of, stored digital assets associated with User Identity.

[R-7.2.4-001] Subject to operator policy, regulatory requirements and user consent, the 5G system shall provide secure means to authorize the use of digital assets associated with a user (e.g., digital assets belonging to a third party customer).

[R-7.2.4-002] The 5G system shall provide mechanisms to certify the authenticity of digital assets associated with a user.

- 4	Detailed proposal

***	BEGIN OF 1st CHANGE***

  - 5.X	Key issue #X: Security aspects of digital asset container in 5GC

    - 5.X.1	Key issue details

According to SA1 TS 22.156 [3], several requirements corresponding the digital asset container of the metaverse are identified:

[R-5.2.3-001] Subject to operator policy, regulatory requirements and user consent, the 5G system shall be able to provide functionality to store digital assets associated with a user, and to remove such digital assets associated with a user.

[R-5.2.3-003] Subject to operator policy and user consent, the 5G system shall be able to allow an authorized third party to retrieve the digital asset(s) associated with a user, e.g., when the user accesses a specific application.

[R-5.2.3-006] Subject to operator policy, regulatory requirements and user consent, the 5G system shall support a mechanism for users to define conditions (e.g., based on user location information) to restrict the access to, and management of, stored digital assets associated with User Identity.

[R-7.2.4-001] Subject to operator policy, regulatory requirements and user consent, the 5G system shall provide secure means to authorize the use of digital assets associated with a user (e.g., digital assets belonging to a third party customer).

[R-7.2.4-002]The 5G system shall provide mechanisms to certify the authenticity of digital assets associated with a user.

The requirements will lead new functionalities of the digital asset container. Correspondingly, the security aspect of the digital asset as well as the digital asset container itself deserve a thorough KI in order to further clarify the potential risks of the digital asset container in 5GC.

This KI aims to comprehensively study the security requirements of the digital asset container in 5GC.

    - 5.X.2	Security threats

Malicious users may spoof the digital asset information and access metaverse services.

Malicious users may impersonate as a legitimate user of the corresponding digital asset.

An attacker may access a digital asset if the authentication and authorisation of the usage of digital assets are not performed.

The potential transfer of the digital asset between networks may be tampered by intermediary network entities.

    - 5.X.3	Potential security requirements

The 5G system shall support securely storing and removing the digital asset(s) in the digital asset container.

The 5G system shall provide the means to ensure authenticity of the digital asset(s) in a digital asset container associated with the user.

The 5G system shall support to authenticate and authorize a 3rd party to access the digital asset(s) in a digital asset container associated with a user.

The 5G system shall support to authenticate and authorize the digital asset(s) in a digital asset container associated with a user.

***	END OF 1st CHANGE	***