3GPP TSG-SA3 Meeting #116 	S3-242097

# Jeju, Korea, 20 - 24 May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0069 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update procedure of UE privacy verification for Ranging/SL positioning service exposure through PC5 link |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The privacy verification procedure of Ranging/SL positioning service exposure through PC5 is incompete. As specified in TS 33.533, the UE1 and UE2/../UEn shall perform UE privacy check, however, how to process the Client UE’s Ranging/SL positioning service request based on the privacy verification result is unresolved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the procedure for UE1 to process the privacy verification result received from UE2/…/UEn. Based on these privacy verification results, the UE1 decides to proceed with or reject the Ranging/SL positioning service request from the Client UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved issues and hence incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Change****

#### 6.3.6.3	Authorization procedure for Ranging/SL positioning service exposure through PC5

For Ranging/SL Positioning service exposure through PC5 (i.e. clause 6.7.1.1 of TS 23.586 [2]), the SL Positioning Client UE authorization is triggered by the Reference/Target UE during PC5 link establishment. The authorization can be performed by the network via the SLPKMF for ProSe capable UEs or by the Reference/Target UE if the authorization information is available in the UE.

For UE-only operation or before triggering SL-MO-LR for Network based operation, the UE1 receiving the Ranging/SL positioning request shall send a supplementary RSPP signalling message to UE2/../UEn to trigger privacy check for Ranging/SL positioning service exposure through PC5. The supplementary RSPP message shall include Client UE's user info ID that is received by UE1 from the Client UE. The UE1 and UE2/../UEn shall perform UE privacy check as described in clause 6.3.7 to determine whether their location related information can be exposed to Client UE. The UE1 and UE2/../UEn shall send the privacy verification result to the UE1. If none of the UEs grants permission for exposure to the Client UE, the UE1 shall reject the Ranging/SL Positioning service request from Client UE. If all of the UEs grant permission for exposure to the Client UE, the UE1 shall proceed with the Ranging/SL positioning service request from the Client UE. If a portion of the UEs grant permission for exposure to the Client UE, and the remaining UEs do not, the UE1 shall proceed with or reject the service request based on its local configuration rule received from UE1’s application layer or the GMLC. To proceed with the service request, the UE1 sends the request to the Server UE for UE-only operation or the LMF for Network based operation, including the Application layer ID of the UEs who have granted permission for exposure to the Client UE.

If the Client UE is not authorized, the Ranging/SL Positioning service request shall be rejected.

**** End of Change****