3GPP TSG-SA3 Meeting #116	S3-242123

Jeju, South Korea,  20th - 24th May 2024

Title:	[Draft] Reply LS on clarification on home network triggered re-authentication

Response to:	LS S3-241736/C4-235577 on LS on clarification on home network triggered re-authentication

Release:	Rel 18

Work Item:	HN_Auth

Source:	Xiaomi [to be SA3]

To:	CT4

Cc:	-

Contact person:	Jouy Shang

shangzhengyi@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall description

SA3 would like to thank CT4 for their LS on Clarification on home network triggered re-authentication. SA3 would like to provide the following feedback.

1. When the (source) AMF receives a Reauth Notifications request, there is an ongoing HO/Mobility registration without AMF change.

2. When the (source) AMF receives a Reauth Notifications request, there is an ongoing HO/Mobility registration with AMF change.

3. When the target AMF receives a Reauth Notifications request, there is an ongoing HO/Mobility registration with AMF change.

4. When the AMF receives a Reauth Notifications request, there is an ongoing authentication.

5. When the AMF receives a Reauth Notifications request, the UE is unreachable and the AMF doesn’t know whether there is another AMF available over the other access type.

Q1. Should the above user cases be considered as valid failure cases?

A1. The cases 2 and 5 can be considered as valid failure cases. For cases 1, 3, and 4, the AMF sends the acknowledgement response to the UDM.

Q2. Does SA3 see any need of differentiated handling in the UDM in each failure case? Or, what’s the expected UDM behaviour from the SA3 point of view?

A2. The UDM should take action as specified in clause 6.1.5 of TS 33.501.

For failure cases 2, the UDM can determine later on whether to trigger the primary authentication by resending Nudm_UECM_Re-AuthenticationNotification message.

For failure case 5, the UDM can check if another AMF is available over the other access and resend the Nudm_UECM_Re-AuthenticationNotification message to the AMF over the other access.

- Actions

To CT4

ACTION: 	SA3 kindly asks CT4 to take the above information into account.

- Date of Next SA3 Meeting

SA3#117	19 - 23 August 2024		Maastricht (Netherlands)

SA3#118	14 - 18 October 2024		TBD (India)