3GPP TSG-SA3 Meeting #116	S3-242154

Jeju, Korea, 20th – 24th May 2024

# Source:	Qualcomm Incorporated

# Title:	A new solution for multi-hop U2N relay communication security

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes a new solution to address the key issue #1.

# 2	References

[1]	Draft TR 33.743 v0.1.0

# 3	Rationale

This contribution proposes a new solution for multi-hop UE-to-Network Relay communication security.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft TR [1].

**** START OF CHANGES ****

## 6.Y	Solution #Y: Multi-hop UE-to-Network Relay communication security

### 6.Y.1	Introduction

This solution addresses the first, second and third security requirements in the key issue #1 regarding the multi-hop UE-to-Network (U2N) Relay communication. This solution proposes to reuse the security procedure over User Plane for PC5 security establishment for each hop among 5G ProSe Remote UE, Intermediate UE-to-Network Relay(s), and 5G ProSe UE-to-Network Relay as specified in clause 6.3.3.2 of TS 33.503 [5]. The proposed security procedure is based on the multi-hop UE-to-Network Relay communication procedures in the solutions (i.e., solution #1, #2, and #7) of TR 23.700-03 [1].

### 6.Y.2	Solution details

The security procedure for multi-hop UE-to-Network Relay communication is shown in Figure 6.Y.2-1.

Figure 6.Y.2-1: Security procedure for multi-hop UE-to-Network Relay communication

0.	The 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay are provisioned with the discovery security materials associated with an RSC based on the procedure specified in clause 6.3 of TS 33.503 [5]. In addition, the 5G ProSe Remote UE and Intermediate UE-to-Network Relay are provisioned with UP-PRUK and UP-PRUK ID from 5G PKMF as specified in step 1 in clause 6.3.3.2.2 of TS 33.503 [5].

1.	The 5G ProSe Remote UE performs a multi-hop UE-to-Network Relay discovery procedure with the Intermediate UE-to-Network Relay and 5G ProSe UE-to-Network Relay.

2.	If the Intermediate UE-to-Network Relay does not have an existing PC5 link with the 5G ProSe UE-to-Network Relay or an intermediate UE-to-Network relay on the path to the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay establishes a PC5 link with the 5G ProSe UE-to-Network Relay or the intermediate UE-to-Network relay based on the PC5 security establishment for 5G ProSe UE-to-Network relay communication over User Plane specified in clause 6.3.3.2.2 of TS 33.503 [5].

3.	The 5G ProSe Remote UE establishes a PC5 link with the Intermediate UE-to-Network Relay based on the PC5 security establishment for 5G ProSe UE-to-Network relay communication over User Plane specified in clause 6.3.3.2.2 of TS 33.503[5] with the Intermediate UE-to-Network Relay taking the role of the 5G ProSe UE-to-Network Relay.

NOTE 1: step 3 can start before step 2.

### 6.Y.3	Evaluation

TBD

**** END OF CHANGES ****