3GPP TSG-SA3 Meeting #116	S3-242156

Jeju, Korea, 20 – 24 May 2024

# Source:	Qualcomm Incorporated

# Title:	A new solution for multihop U2U relay communication security

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes a new solution to address the key issue #2.

# 2	References

[1]	Draft TR 33.743 v0.1.0

# 3	Rationale

This contribution proposes a new solution for multi-hop UE-to-UE Relay communication security.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft TR [1].

**** START OF CHANGES ****

## 6.Y	Solution #Y: Multi-hop UE-to-UE Relay communication security

### 6.Y.1	Introduction

This solution addresses the first, second, and third security requirements in the key issue #2 regarding the multi-hop UE-to-UE (U2U) Relay communication. This solution assumes the architecture and procedures proposed in the solution #3 of TR 23.700-03 [1]. This means that UE-to-UE Relays establish a PC5 link with each other after the Relay discovery to form a 5G ProSe UE-to-UE Relay cloud, and a 5G ProSe End UE establishes a PC5 link with the 5G UE-to-UE Relay to discover a target 5G ProSe End UE and communicate with the discovered target 5G ProSe End UE at IP layer (e.g., based on MANET routing protocol).

For PC5 link security, this solution proposes to reuse the security procedure specified in clause 6.2 of TS 33.503 [5].

### 6.Y.2	Solution details

Based on the architecture and procedures in the solution #3 of TR 23.700-3 [1], this solution consists of two types of PC5 link establishment: one for PC5 link establishment among 5G ProSe UE-to-UE Relays and the other one for PC5 link establishment between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay.

1.	PC5 link establishment among 5G ProSe UE-to-UE Relays:

After 5G ProSe UE-to-UE Relays perform a Relay discovery, they establish a secure PC5 link based on the security procedure for unicast mode 5G ProSe Direct Communication specified in clause 6.2 of TS 33.503 [5] with the following modifications:

-	The RSC is included in the DCR message.

-	The DCR message is protected based on the security mechanism defined in clause 6.3.5 with a modification that the UP-PRUK ID/CP-PRUK ID is not used in clause 6.3.5.2.

2. PC5 link establishment between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay:

After the Relay discovery between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay, the 5G ProSe End UE establishes a secure PC5 link with the 5G ProSe UE-to-UE Relay based on the security procedure for unicast mode 5G ProSe Direct Communication specified in clause 6.2 of TS 33.503 [5] with the following modifications:

-	The RSC is included in the DCR message.

-	The DCR message is protected based on the security mechanism defined in clause 6.3.5 with a modification that the UP-PRUK ID/CP-PRUK ID is not used in clause 6.3.5.2.

### 6.Y.3	Evaluation

TBD

**** END OF CHANGES ****