3GPP TSG-SA3 Meeting #116	S3-242208

Jeju, South Korea,  20th - 24th May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.527 | CR | 0005 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Case for Intra-VNF Communication Interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Monolithic NFs communicate using inter-process communication means. One challenge when moving to microservices-based NF is the communication mechanism between microservices of a NF. Because microservices are distributed and should communicate with each other by inter-service communication on network level. VNFCIs belonging to the same VNF could be developed and deployed using an microservices architecture. Security of VNFCIs communication needs to be tested. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Missing test case for intra-VNF (inter VNFCI) communication interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******Start of changes*****

### 4.3.6.x	Intra VNF communication

Requirement Name: secure intra-VNF communication

Requirement Description:

The interface used for the communication between the VNFCIs (VNF Component Instances) of a VNF (intra-VNF traffic) shall be authenticated and protected to prevent the security threats related to insecure network communications.

Threat Reference: Security threat caused by insecure network services in clause 5.3.2.7.10 of TR 33.927 [3]

Test case:

Test Name: TC_SECURE_INTRA-VNF_COMMUNICATION

Purpose:

To test whether the interface between VNFCIs of a VNF is authenticated and protected.

Procedure and execution steps:

Pre-Condition:

1.	There is a VNF instance on the test environment. This VNF instance has more than one VNFCI. The network between VNFCIs means intra-VNF network which is private network provided by vendor.

2.	The document which describes the mechanism used to enable one VNFCI of the VNF to communicate with the other VNFCIs of that VNF.

Execution Steps:

- The tester checks that the interface between VNFCIs of a VNF is authenticated and protected

Expected Results:

In the step 1, interface is authenticated and protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

******End of changes*****