3GPP TSG-SA3 Meeting #116	S3-242248

Jeju, South Korea, 20th - 24th May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0075 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of local privacy check in clause 6.3.7 for server UE request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the UE-only procedure as specified in clause 6.8 of TS 23.586, in steps 9 and 10, a SL Positioning Server UE can request the location information of other UEs, via issuing an RSPP supplementary services request via UE1 (i.e. Target UE). This is very similar to exposure of location information to a SL Positioning Client UE, and could even impose additional privacy risk since a SL Positioning Server UE may request for additional information such as measurements from other UEs. Therefore, the local privacy check for UE-only operation needs to take into account that the location is meant to be shared with the SL Positoning Server UE, and not just peer UE and/or SL Positioning Client UE. This is because it may reveal privacy sensitive location information of the UE to a SL Positioning Server UE, which the UE may not agree to. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add details to clause 6.3.7 on how a local privacy check is performed in case location information is exposed to a SL Positioning Server UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Privacy check of exposing the location information to a SL Positioning Server UE in UE-only operation is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3.7	Procedure of UE privacy verification for UE-only operation

For UE-only Operation in which the network is not involved in Ranging/Sidelink positioning, the authorization for UE privacy is based on the local configured privacy verification information to determine whether its location related information can be exposed to the peer UE or not. If the privacy profile allows location exposure, the UE (e.g. Located UE) accepts the request to expose its location related information and proceeds.

To enable privacy check of exposure to a SL Positioning Client UE or SL Positioning Server UE via the peer UE, the peer UE shall include the user info ID of the SL Positioning Client UE respectively of the SL Positioning Server UE in the supplementary RSPP signalling message to the UE from which the location information or related results are to be exposed.