3GPP TSG-SA3 Meeting #116	S3-242350

Jeju, South Korea,  20th - 24th May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | 0198 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support cleartext HPLMN ID in PC5 U2N relay discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to clause 6.1.3.2.2 of the present TS, PROSE PC5 DISCOVERY message for UE-to-network relay discovery includes HPLMN ID in cleartext to identify the discovery security materials. the ProSe U2N remote UE can obtain the proper security material in order to unscramble and decrypt the message based on the HPLMN ID. However, based on existing encryption and scrambling operations defined in 33.503, the HPLMN ID may be scrambled and probably encrypted. If so, ProSe remote UE cannot decide which security material/key should be used to un-scramble or decrypt the message, and have to retry the unscrambling and decryption multiple times i.e. with different security materials till it reaches a match for the HPLMN ID, that is time and energy consuming. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update scrambling and encryption operations to support clear text HPLMN ID in the U2N relay discovery message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ProSe U2N relay discovery over PC5 may be failure or take long time |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

################### Start the 1st change ##################

##### 6.1.3.2.3	Protection of discovery messages over PC5 interface

There are three types of security that are used to protect the restricted 5G ProSe Direct Discovery messages over the PC5 interface: integrity protection, scrambling protection, and message-specific confidentiality which are defined in clause 6.1.3.4.3 in TS 33.303 [4]. The protection mechanisms specified in TS 33.303 [4] are reused with the following changes:

-	Input parameters to integrity protection algorithm as specified in clause A.6 in the present document.

-	Message-specific confidentiality mechanisms as specified in clause A.7 in the present document. For 5G ProSe UE-to-Network Relay discovery, the HPLMN ID shall not be encrypted. KEYSTREAM for message-specific confidentiality protection shall be updated accordingly to support non-encryption of HPLMN ID

-	In A.5 of TS 33.303 [4], the time-hash-bitsequence keystream is set to L least significant bits of the output of the KDF, where L is the bit length of the discovery message to be scrambled and set to Min (the length of discovery message - 16, 256).

-	Step 3 of clause 6.1.3.4.3.5 of TS 33.303 [4] becomes:

XOR (0xFFFF || time-hash-bitsequence) with the most significant (L + 16) bits of discovery message.

NOTE 1:	16 is the size of Message Type and UTC-based counter LSB in bit length.

NOTE 2:	The maximum length of the discovery message to be scrambled is limited to 256 bits.

NOTE x:  For 5G ProSe UE-to-Network Relay discovery, the HPLMN ID shall not be scrambled. time-hash-bitsequence shall be updated accordingly to support non-scrambling of HPLMN ID.

-	Step 2 of clause 6.1.3.4.3.2 of TS 33.303 [4] becomes:

Calculate MIC if a DUIK was provided, otherwise set MIC to a 32-bit random string. Then, set the MIC IE to the MIC.

-	Step 4 of clause 6.1.3.4.3.2 of TS 33.303 [4] is not processed.

NOTE 3: Protection for the discovery messages between the ProSe UEs is provided at the ProSe layer.

################### End the change ##################