3GPP TSG-SA3 Meeting #116	S3-242353

Jeju, South Korea,  20th - 24th May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | 0200 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix issue of scrambling operation for PC5 ProSe discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 6.1.3.2.3 states the following (related to the message scrambling): - In A.5 of TS 33.303 [4], the time-hash-bitsequence keystream is set to L least significant bits of the output of the KDF, where L is the bit length of the discovery message to be scrambled and set to Min (the length of discovery message - 16, 256). - Step 3 of clause 6.1.3.4.3.5 of TS 33.303 [4] becomes: XOR (0xFFFF ¦¦ time-hash-bitsequence) with the most significant (L + 16) bits of discovery message. NOTE 1: 16 is the size of Message Type and UTC-based counter LSB in bit length. According to scrambling operation described above, if L is less than or equal to 256, the whole discovery message is XORed with (0xFFFF ¦¦ time-hash-bitsequence), and finally MIC and payload are scrambled. If L is more than 256, e.g. in Figure 3-1, if RRC container IE is present and its length is 257, as only the most significant (L + 16) bits of discovery message is XORed with (0xFFFF ¦¦ time-hash-bitsequence), all mandatory IEs in the payload and MIC will not be scrambled, that may cause privacy issue.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update scrambling operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The sensitive information in the PC5 ProSe discovery message may not be protected and cause privacy issue |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

################### Start the 1st change ##################

##### 6.1.3.2.3	Protection of discovery messages over PC5 interface

There are three types of security that are used to protect the restricted 5G ProSe Direct Discovery messages over the PC5 interface: integrity protection, scrambling protection, and message-specific confidentiality which are defined in clause 6.1.3.4.3 in TS 33.303 [4]. The protection mechanisms specified in TS 33.303 [4] are reused with the following changes:

-	Input parameters to integrity protection algorithm as specified in clause A.6 in the present document.

-	Message-specific confidentiality mechanisms as specified in clause A.7 in the present document.

-	In A.5 of TS 33.303 [4], the time-hash-bitsequence keystream is set to L least significant bits of the output of the KDF, where L is the bit length of the discovery message to be scrambled and set to Min (the length of discovery message - 16, 256).

-	Step 3 of clause 6.1.3.4.3.5 of TS 33.303 [4] becomes:

XOR (0xFFFF || time-hash-bitsequence) with the least significant (L + 16) bits of discovery message.

NOTE 1:	16 is the size of Message Type and UTC-based counter LSB in bit length.

NOTE 2:	The maximum length of the discovery message to be scrambled is limited to 256 bits.

-	Step 2 of clause 6.1.3.4.3.2 of TS 33.303 [4] becomes:

Calculate MIC if a DUIK was provided, otherwise set MIC to a 32-bit random string. Then, set the MIC IE to the MIC.

-	Step 4 of clause 6.1.3.4.3.2 of TS 33.303 [4] is not processed.

NOTE 3: Protection for the discovery messages between the ProSe UEs is provided at the ProSe layer.

################### End the change ##################