3GPP TSG-SA3 Meeting #116	S3-242410

Jeju, South Korea,  20th - 24th May 2024										revision of S3-242345

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | 0196 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add clarification on encryption operation for PC5 ProSe discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Annex A.7 states the following (related to the message encryption): Message-specific confidentiality protection is provided by ProSe layer between ProSe UEs. The use and mode of operation of the ciphering algorithms are specified in Annex D in TS 33.501 [3]. The input parameters to the ciphering algorithms as described in Annex D in TS 33.501 [3] are: - KEY: 128 least significant bits of the output of the KDF (DUCK, UTC-based counter, MIC) - COUNT: UTC-based counter - BEARER: 0x00 - DIRECTION: 0x00 - LENGTH: LEN(discovery message) - (LEN(Message Type) + LEN(UTC-based counter LSB) + LEN(MIC)), where LEN(x) is the length of x in number of bits KEY is set to as such to generate message-specific keystream as in TS 33.303 [4]. The output keystream of the ciphering algorithm (output_keystream) is then masked with the Encrytped_bits_mask to produce the final keystream for the message-specific confidentiality protection (KEYSTREAM): KEYSTREAM = output_keystream AND (Encrypted_bits_mask ¦¦ 0xFF..FF) The KEYSTREAM is XORed with the discovery message for message-specific confidentiality protection. It's not clear which part of the discovery message will be XORed with the KEYSTREAM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification on encryption operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The parameter of the operation is not clear which may cause inconsistent result and cause discovery failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

################### Start the 1st change ##################

# A.7	Message-specific confidentiality mechanisms for discovery

Message-specific confidentiality protection is provided by ProSe layer between ProSe UEs.

The use and mode of operation of the ciphering algorithms are specified in Annex D in TS 33.501 [3].

The input parameters to the ciphering algorithms as described in Annex D in TS 33.501 [3] are:

-	KEY: 128 least significant bits of the output of the KDF (DUCK, UTC-based counter, MIC)

-	COUNT: UTC-based counter

-	BEARER: 0x00

-	DIRECTION: 0x00

-	LENGTH: LEN(discovery message) - (LEN(Message Type) + LEN(UTC-based counter LSB) + LEN(MIC)), where LEN(x) is the length of x in number of bits

KEY is set to as such to generate message-specific keystream as in TS 33.303 [4].

The output keystream of the ciphering algorithm (output_keystream) is then masked with the Encrytped_bits_mask to produce the final keystream for the message-specific confidentiality protection (KEYSTREAM):

KEYSTREAM = output_keystream AND (Encrypted_bits_mask || 0xFF..FF)

The KEYSTREAM is XORed with the discovery message for message-specific confidentiality protection excluding Message Type, UTC-based counter LSB and MIC.

################### End the change ##################