3GPP TSG-SA3 Meeting #117	S3-242862

Maastricht, Netherlands, 19th – 23rd August 2024

# Source:	MITRE-FFRDC, Defense Information Systems Agency EM

# Title:	New solution for KI#1: Analytics for Secure N4 interface with Security Gateway

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This pCR proposes a solution for Key Issue #1 in TR 33.757[1]: Security for dedicated UPF interacting with PLMN through N4 interface

# 2	References

[1]			3GPP TR 33.757 Study on security for PLMN hosting a NPN.

[2]		3GPP TR 33.794, 'Study on enablers for Zero Trust Security', (Release 19).

# 3	Rationale

This solution addresses TR 33.757 [1] KI#1 (Key Issue #1: Security for dedicated UPF interacting with PLMN through N4 interface). The proposed solution leverages existing work to add additional functionality to solutions for Key Issue #1 to perform analytics on the dedicated UPF.

# 4	Detailed proposal

SA3 is kindly requested to approve the below change to TR 33.757 [1]

********************** Start of 1st Change ****************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G system"

[4]            3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[5]            3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes"

[6]            3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)"

[7]            IETF RFC 4303: "IP Encapsulating Security Payload (ESP)"

[8]	3GPP TS 23.273: " 5G System (5GS) Location Services (LCS); Stage 2"

[9]	3GPP TS 23.501: " System architecture for the 5G System (5GS); Stage 2"

[10]           3GPP TS 29.500: "Technical Realization of Service Based Architecture"

[11]	3GPP TS 33.126: " Lawful Interception requirements"

[12]           3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security"

[AA]	3GPP TR 33.794, 'Study on enablers for Zero Trust Security', (Release 19).

********************** End of 1st Change ****************************

********************** Start of 2nd Change ****************************

## 7.Y	Solution #Y: Analytics for Secure N4 interface with Security Gateway

### 7.Y.1	Introduction

This solution addresses key issue #1. The proposed solution leverages existing work to add functionality to solutions for Key Issue #1 to perform analytics on the dedicated UPF.

### 7.Y.2 Solution Details

Solutions to Key Issue #1: Security for dedicated UPF interacting with PLMN through N4 interface only cover aspects on how to perform security enforcement between PLMN operational domain and the PNI-NPN operational domain. However, the solutions do not describe how the operator can monitor data over time for errors and/or malicious N4 messages and perform analytics on that data. Therefore, this solution proposes to introduce an extension to solutions addressing key issue#1 to allow the PLMN operator and/or PNI-NPN customer to collect data from filtered traffic and use analytics to determine malicious intent.

To perform analytics this solution proposes leveraging the data collection mechanisms proposed in the FS_eZTS study TR 33.794 [AA] Key Issue #1: Data exposure for security evaluation and monitoring. For example, the security gateway(s) lying between the PLMN operational domain and the PNI-NPN operational domain can send malformed messages to the operator security function to perform analytics, then based upon the analytics the operator will return updated security policy to enforce at the security gateway accordingly.

#### 7.Y.2.1 Relevant Data for Collection

The data which may be relevant for threats and attack detection between the PLMN operational domain and the PNI-NPN operational domain is the messages on the N4 interface that are either filtered to prevent topology discovery or malformed messages, as described in Key Issue #1 Security Threats.

In addition, when relevant, the data identified in FS_eZTS study TR 33.794 [AA] clause 5.1 may be collected by the operator security function. For example, collecting identifiers when authentication fails between operator premise dedicated UPF and the security gateway.

### 7.Y.3 Evaluation

The proposed solution leverages existing work to add functionality to solutions for Key Issue #1 to allow analytics to be performed on N4 interface data.

********************** End of 2nd Change ****************************