3GPP TSG-SA3 Meeting #117	S3-242868

Maastricht, Netherlands, 19th – 23rd August 2024

# Source:	MITRE-FFRDC, US National Security Agency

# Title:	Conclusions for Data Collection

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

This pCR proposes conclusions for TR 33.794[1] to specify the specific data collected for security monitoring and evaluation.

# 2	References

[1]	3GPP TR 33.794 Study on enablers for Zero Trust Security

[2]	3GPP TS 29.510 5G System; Network function repository services; Stage 3

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

# 3	Rationale

The present document studies the use cases for security evaluation and monitoring in clause 5.1. This study identified and evaluated the relevant data to be collected to aid in security evaluation and monitoring for each use case. However, in some cases it was up to the conclusions to determine the specific data for collection. Therefore, this conclusions document proposes a set of data for collection.

# 4	Detailed proposal

SA3 is kindly requested to approve the below change to TR 33.794 [1]

********************** Start of 1st Change ****************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 33.894, 2023 September, V18.0.0: "Study on applicability of the zero trust security principles in mobile networks", Release 18.

[3]	3GPP SP-231784, "New Study on enablers for Zero Trust Security".

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[5]	RFC 6749, "The OAuth 2.0 Authorization Framework".

[6]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[7]	3GPP TR 33.894, 2023 September, V18.0.0: "Study on applicability of the zero trust security principles in mobile networks", Release 18.

[8]	NIST Special Publication 800-207: "Zero Trust Architecture".

[9]	3GPP TR 33.738: "Study on security aspects of enablers for network automation for the 5G system phase 3".

[10]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[11]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[12]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[13]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[14]	IETF RFC 9113: "HTTP/2".

[15]	3GPP TS 33.117: "Catalogue of general security assurance requirements"

[16]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes

[17]	https://owasp.org/www-community/Threat_Modeling_Process

[18]	3GPP TS 23.501: " System architecture for the 5G System (5GS)".

[19]	NIST SP-800-92: "Guide to Computer Security Log Management".

[20]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[AA]	IETF RFC 9110: "HTTP Semantics".

[BB]	https://www.misp-project.org/datamodels/.

********************** End of 1st Change ****************************

********************** Start of 2nd Change ****************************

# 8	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 8.X Data collection

The following table identifies data for collection to aid in security monitoring and evaluation identified in clause 5.1.

Table 8.X-1: Security event data attributes

| Information | Description |
| --- | --- |
| Security event type | This IE indicates the type of security incident that triggers this event notification. (e.g., authentication failure, authorization failure, malformed message, flooding) |
| Timestamp | This IE contains the observed time of the event. |
| Target NF instance ID | This IE contains the NF instance ID of the target NF. |
| HTTP status code | This IE contains the HTTP status code as defined in IETF RFC 9110 [AA]. This is the HTTP status code that the target NF returns to the source NF in a given SBA API call. |
| Source network info | This IE contains the network information of the source NF. Network information may include src/dst IP address, src/dst port, total packets sent, total bytes sent, first/last packet seen, and transport protocol. The format for these attributes may follow well known data models such as MISP [BB]. |
| Source NF instance ID | This IE contains the NF instance ID of the source NF. |
| Source service request | This IE contains the service(s) requested (e.g., API name) by the source NF. |
| Source PLMN ID | This IE contains the PLMN ID of the source NF. |
| Source TLS certificate | This IE contains the TLS certificate including domain name, CA, expiration/issuance date of the source NF. |
| Source API invocation history | This IE contains information regarding past service requests from the source NF (e.g., service request, authentication/authorization status, timestamps). This data relies on storage of past events. |
| AccessTokenClaims | As defined in clause 6.3.5.2.4 of TS 29.510 [20]. |
| NF resource usage | As defined in clause 6.5 of TS 23.288 [13] |
| Status, load and performance information | As defined in clause 6.6 of TS 23.288 [13] |
| NOTE 1: The source NF is the NF originating the communication with the target NF. The target NF is the NF receiving the communication request. |  |

********************** End of 2nd Change ****************************