3GPP TSG-SA3 Meeting #117	S3-242888

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Sol#21 update to address ENs

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Approve the pCR proposal for TR 33.700-32

# 2	References

[1]	3GPP TR 33.700-32

# 3	Rationale

This contribution proposes text to address 5 ENs for Sol#21 in TR 33.700-32 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 6.21	Solution #21: A&A of non-3GPP devices behind UE or 5G-RG based on secondary authentication

### 6.21.1	Introduction

This solution addresses the key issue #3.

The solution reuses the secondary authentication procedure in TS 33.501 [3] and adapts it to authenticate and authorize a non-3GPP device behind a UE or 5G-RG.

### 6.21.1	Solution details

Assuming the non-3GPP device behind a UE is identified by the Device IDentifier (DID) linked to a UE, with a 3GPP subscription identified by the UE’s SUPI/GPSI, to access services via the 5GS. Before authentication of the DID, the Primary authentication for the UE needs to be performed.

NOTE 1: the DID is assigned by the non-3GPP system. The assignment is out of scope of 3GPP.

With reference to the figure 11.1.2-1 in TS 33.501 [3], the DID authentication and authorization procedure is described as follows:

1-3. UE is registered to the network after Primary authentication and security context is established as in TS 33.501 [3].

4-7. The UE sends a PDU session establishment request to the network as in TS 33.501 [3]. The PDU session establishment request may include the DID of the non-3GPP device similar to the procedure in the TS 33.501 [3] (as in the step 10 of the clause 11.1.2).

8. The H-SMF initiates the device authentication procedure for the PDU session as in TS 33.501 [3]. The SMF may trigger the device authentication based on the UE subscription information.

NOTE 2: as in the TS 33.501 [3], the SMF can trigger based on UE subscription information or the DID sent from the UE.

9-13. The EAP authentication starts and is completed after multiple rounds of message exchanges between the UE and the DN, as in TS 33.501 [3].

The EAP Response/Identity message sent to the DN may include the GPSI of the UE.

14. After the successful completion of the authentication procedure, DN AAA server shall send an EAP Success message to the H-SMF, along with the GPSI and the DID. The message may include any restriction information imposed to the DID, e.g. tiers of services/QoS, service duration etc.

The DN AAA server can identify the DID based on the GPSI and/or the EAP ID. The DID may be the same as the EAP ID or may be different from the EAP ID. If the DID is different from the EAP ID, the DN AAA server should be provisoned with the mapping between the DID and the EAP ID.

15. This completes the authentication procedure at the SMF. The SMF may save the DN-specific ID and DNN (or DN's AAA server ID if available) in a list for successful authentication/authorization between UE and an SMF. Alternatively, the SMF may update the list in UDM. The UE (and the non-3GPP device) is identified by the GPSI and DID.

If the authorization is successful, PDU Session Establishment proceeds as described in TS 33.501 [3].

16a-19 The UE-requested PDU Session Establishment authentication/authorization by a DN-AAA server proceeds further as described in TS 33.501 [3].

### 6.21.3	Evaluation

***  END OF CHANGES  ***