3GPP TSG-SA3 Meeting #117	S3-242889

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Sol#21 evaluation

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Approve the pCR proposal for TR 33.700-32

# 2	References

[1]	3GPP TR 33.700-32

# 3	Rationale

This contribution proposes evaluation text for Sol#21 in TR 33.700-32 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 6.21	Solution #21: A&A of non-3GPP devices behind UE or 5G-RG based on secondary authentication

### 6.21.1	Introduction

This solution addresses the key issue #3.

The solution reuses the secondary authentication procedure in TS 33.501 [3] and adapts it to authenticate and authorize a non-3GPP device behind a UE or 5G-RG.

### 6.21.1	Solution details

Assuming the non-3GPP device is identified as DN identifier (DID) behind a UE with a 3GPP subscription identified by the UE’s SUPI/GPSI to access services via the 5GS. Before authentication of the DID, the Primary authentication for the UE needs to be performed.

Editor’s note: The relation between DID and non-3GPP device identifier is FFS.

With reference to the figure 11.1.2-1 in TS 33.501 [3], the DID authentication and authorization procedure is described as follows:

1-3. UE is registered to the network after Primary authentication and security context is established as in TS 33.501 [3].

4-7. The UE sends a PDU session establishment request to the network as in TS 33.501 [3].

8. The H-SMF initiates the secondary authentication procedure for the PDU session as in TS 33.501 [3]

9-13. The EAP authentication starts and is completed after multiple rounds of message exchanges between the UE and the DN, as in TS 33.501 [3].

14. After the successful completion of the authentication procedure, DN AAA server shall send EAP Success message to the H-SMF, along with GPSI and DID. The message may include any restriction information imposed to the DID, e.g. tiers of services/QoS, service duration etc.

Editor’s note: How the DN AAA server gets aware of the linked relation between GPSI of the UE and DID of the non-3GPP device is FFS.

15. This completes the authentication procedure at the SMF. The SMF may save the DN-specific ID and DNN (or DN's AAA server ID if available) in a list for successful authentication/authorization between UE and an SMF. Alternatively, the SMF may update the list in UDM. The UE (and the non-3GPP device) is identified by the GPSI and DID.

If the authorization is successful, PDU Session Establishment proceeds as described in TS 33.501 [3].

16a-19 The UE-requested PDU Session Establishment authentication/authorization by a DN-AAA server proceeds further as described in TS 33.501 [3].

### 6.21.3	Evaluation

Editor’s note: Without indication of Device ID from UE (steps 4-7 and 8), how the SMF determines to trigger the authentication is FFS

Editor’s note: How secondary authentication of the UE authenticates the non-3GPP device behind the UE is FFS.

Editor’s note: ffs to clarify DID.

This solution addresses the key issue #3.

The solution reuses the secondary authentication procedure and adapts it to authenticate and authorize a non-3GPP device behind a UE or 5G-RG. It uses the EAP framework so that various authentication methods can be supported.

***  END OF CHANGES  ***