3GPP TSG-SA3 Meeting #117	S3-242890

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	conclusions for KI#3

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Approve the pCR proposal for TR 33.700-32

# 2	References

[1]	3GPP TR 23.700-32 Study on User Identities and Authentication Architecture

# 3	Rationale

This contribution proposes normative work to address KI#3, in line with SA2 conclusions in the TR 23.700-32 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 7.3	Conclusions to Key Issue #3

Normative work is recommended to support authentication and authorization of a non-3GPP device behind a UE or a 5G-RG based on the non-3GPP device identifier, following the principles below:

-The procedure of authenticataion and authorization for the non-3GPP device by a third party is a PDU-session based method, and similar to Secondary Authentication in Clause 11 of TS 33.501 [3].

-The UE/5G-RG sends the non-3GPP device identifier to an SMF in a NAS-SM message, and information corresponding to the authorized non-3GPP device identifier can be stored at the UDR/UDM.

In addition, the following principles are assumed:

-Authentication of the non-3GPP device is not done by the 5GC.

-The non-3GPP device identifier is a permanent identifier and the allocation of the identifier is out of scope of 3GPP.

-The UE/5G-RG will bind the device identifier to a non-3GPP device and how to perform binding is out of scope of 3GPP.

Editor’s Note: The above conclusions can be updated in line with SA2 progress.

***  END OF CHANGES  ***