3GPP TSG-SA3 Meeting #117	S3-242903

Maastricht, Netherlands  19 - 23 August 2024

Title:	draft - Reply LS on User Identities and Authentication Architecture

Response to:	LS S2-2407236/S3-242724 on User Identities and Authentication Architecture from SA2

Release:	Rel-19

Work Item:	FS_UIA_ARC

Source:	ZTE Corporation (to be SA3)

To:	SA2

Cc:	SA3-LI, SA1

Contact person:	Peilin Liu

liu.peilin@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

SA3 would like to thank SA2 for their LS S2-2407236/S3-242724 on User Identities and Authentication Architecture.

SA3 would like SA2 to consider following answers.

SA2 Question1a:

During the discussion related to Key Issue #1 and Key Issue #2 most companies believe that it is appropriate to send the user identifier in NAS (e.g., to trigger authentication by the network and to activate the user with a subscription). Different from the majority view, some companies objected to this conclusion and raised the issue of trust of the user identifier and credentials for the MNO (HPLMN) and questioning whether the HPLMN will be able to trust the user identifier and credentials.

While considering that the views from majority of companies being,

- the user identity and credentials that will be used to identify the user in HPLMN might not be stored in UICC. For example, a User Interface could be used to retrieve user identifier and credentials directly from the human user; or the User Interface could identify the human (e.g., based on biometric authentication) and related human user identifier and credentials could be retrieved by the operating system (e.g., from the UICC or other local storage).

- procedures and methods similar to NSSAA will be used to authenticate the user.

SA2 requests SA3 to provide feedback if their views are inline to the majority view as described above.

Answer to Question1a:

SA3 understands that user identifier is linked with SUPI under MNO control, and prefers sending user identifier in NAS established using UE credentials. If the user credentials is used directly, the MNO cannot guarantee the use of the right user identifier and credentials, which will be out of MNO control and causing trust issue. SA3 has an Key Issue #1 on the Authentication and Authorization of Human User ID and would keep SA2 informed of progress.

SA2 Question1b.

Regarding the Key Issue #3 of TR 23.700-32, can any existing information of the UE subscription with whom the user identifier is active, or new information related to the user to be allowed to be exposed to an authorized AF, and if so, is user consent required for the exposure of this information? Specifically, we would like to understand the user consent requirements for the following new information:

- the verification result indicating whether the User Identifier authentication is successful.

Answer to Question1b:

SA3 thinks the exposure of user related information needs the consent from the user whom the user identifier is active. If the user consents the exposure of user related information, the authorized AF can acquire the information form the core network.

SA2 Question2:

Regarding Key Issue #1 of TR 23.700-32, SA2 kindly requests SA3 to provide their opinion whether there is a privacy issue if the network keeps providing SMS and IMS services to a UE when a user identifier is active with the UE subscription.

Answer to Question2:

SA3 thinks this depends on the requirements of SA1. If SA1 agrees that there are privacy requirements on SMS and IMS services to a UE when a user identifier is active with the UE subscription, SA3 would like to work on the security mechanism to address the issues.

2	Actions

To SA2

ACTION: 	SA3 would like to respectfully ask SA2 to take above answers into consideration.

3	Dates of next TSG SA WG 3 meetings

SA3#118	14 - 18 Oct 2024		India, IN

SA3#119	11 - 15 Nov 2024		Orlando, US